Compilation of

Submitted Inputs

to the Progress Report

to the Fifth North Sea Conference

__________________________________________________________________________

Further Reduction of

Nutrient Inputs to the North Sea

The North Sea Secretariat

January 2002

Reports submitted on further reduction of nutrient inputs to the North Sea

Introduction

In order to collect information for the Progress Report to the 5th North Sea Conference the secretariat circulated an electronic reporting format (HARP-NUT) to North Sea states in December 2000. Requests for reporting on other action points from the Esbjerg Declaration was sent to the relevant countries and organizations in March 2001. This document comprises responses received.

CONSSO and the OSPAR Eutrophication Committee (EUC) decided to use data from the year 2000 for reporting on the 50 % reduction target. The latest deadline for submitting national reports was 1 December 2001.

Reports regarding the other action points from the Esbjerg Declaration have been received from all relevant countries. The quality of the submitted reports is mostly satisfactory. The responses are structured according to reporting body and paragraphs in the Esbjerg Declaration chapter IV (in square brackets). These reports are presented in Part I of this document.

National reports on the 50 % reduction target have been received from all countries. The reports are presented in Part II of this document.

List of Content

Part I: Inputs received on action points from the Esbjerg Declaration	..2
1.  Sensitive areas/Urban Waste Water Directive/Nitrate Directive	.4
2.  Agricultural policies	10
3.  Balanced fertilisation	13
4.  Atmospheric deposition and ammonia emission	.15

Part II: Inputs of nutrients using the HARP-NUT Guidelines	24
1.  France	25
2.  UK	…31

3.  Belgium……………………………………………………………………………………41

4.  Denmark…………………………………………………………………………………...47

5.  Germany…………………………………………………………………………………...66

6.  Norway…………………………………………………………………………………….89

7.  Sweden…………………………………………………………………………………...104

8.  Switzerland……………………………………………………………………………….122

9.  The Netherlands………………………………………………………………………….129

Compilation of Submitted Inputs

Further reduction of nutrient inputs to the North Sea

Part I:

Inputs received on action points from

the Esbjerg Declaration

Table 1: Inputs received on action points from the Esbjerg Declaration

| Issue | Reference to article in ED | Reporting body | Remarks |
| --- | --- | --- | --- |
| Sensitive areas Urban Waste Water | 31 i | France |  |
|  |  | Norway |  |
|  |  | United Kingdom |  |
|  | 31 ii | European Commission |  |
|  | 31 iii | European Commission |  |
| Agricultural policies | 36 i and ii | European Commission |  |
|  |  | Norway |  |
|  |  | Switzerland |  |
| Balanced fertilisation | 37 | OSPAR |  |
| Atmospheric deposition | 38 i | European Commission |  |
|  |  | Norway |  |
|  |  | Switzerland |  |
|  | 38 ii | UN-ECE |  |
| Ammonia emissions | 38 iii | OSPAR |  |
| Transparent reporting | 67 | European Environmental Agency |  |
|  |  | European Commission |  |
|  |  | Norway | HARP-NUT Guidelines |

1.	Sensitive areas/Urban Waste Water Directive/Nitrates Directive

Therefore the Ministers AGREE to remain committed to reach the reduction targets set by the previous Conferences and to strengthen the implementation of measures as soon as possible. Fundamental elements in fulfilling these goals in the EU and European Economic Area are, inter alia:

i.	to apply in the North Sea and its catchment the measures for sensitive areas under the Urban Waste Water Directive and to apply the measures for vulnerable zones under the conditions of the Nitrates Directive, including the criterion of contribution to pollution as mentioned in Article 3 of the Nitrates Directive.

These measures will be implemented for the whole North Sea and its catchment, except for those parts of the North Sea where comprehensive scientific studies, to be delivered by 1997, demonstrate to the satisfaction of the Committees set up under the respective Directives or the relevant European Economic Area body that nutrient inputs do not cause eutrophication effects or contribute to such effects in other parts of the North Sea.

It is expected that these measures will, in newly identified areas, be applied as soon as technically possible after 1998;

ii.	full implementation of Council Directive 91/271/EEC on Urban Waste Water Treatment in accordance with the timetable stipulated therein or as amended in the future; and

iii.	action to achieve the reduction for nitrate losses from agriculture under the national action plans required by Council Directive 91/676/EEC. These states will take complementary action necessary to meet the commitment in coordination with these action plans.

In parallel, Member States to EU will endeavour to optimise the application of existing EU wide provisions, i.e. Regulation 2078/92 on agri-environmental measures and Regulation 1756/92 on set-aside schemes, to this end. [31]

European Commission:

Urban Waste Water Treatment Directive (91/271/EEC)

The Directive on waste water treatment, which was adopted by the Council in 1991, aims to protect the environment from the adverse effects of the discharge of urban wastewater biodegradable waste water from the food-processing industry. The main obligation imposed by the Directive concerns the establishment of waste water collection and treatment systems.

Ten years after the adoption of this directive, the vast majority of Member States show major delays and shortcomings in its implementation. 37 large cities are still discharging untreated waste water into the environment, and many others are discharging large quantities of effluent after inadequate treatment. Almost all Member States are very slow in providing the Commission with information about the treatment of city sewage in their country.

Commenting on the importance of the Directive Commissioner Wallström said: “The environment of the EU would look different if legislation was enforced in Member States. Waste water treatment, for example, makes the difference between good and bad bathing water and thereby directly affects people’s health.“ She continued: “Member States are also setting a bad example and sending a deplorable message to the candidate countries, which are being criticised for not transposing EU environmental legislation quickly enough.”

Most Member States fail to implement

There are three major deadlines in the Directive.

- The first was 31 December 1998 by which date all agglomerations with more than 10,000 inhabitants discharging their effluent into or upstream of an area regarded as sensitive should have been equipped with a stringent collection and treatment system.

- By the end of 2000 – nine years after adoption – Member States should have established secondary collection and treatment systems (usually biological treatment) in agglomerations with more than 15,000 inhabitants.

- The final deadline is 31 December 2005 for the establishment of appropriate secondary treatment in small agglomerations.

Most Member States have major shortcomings with regards with all these deadlines and with the identification of sensitive areas.

- Major delays in implementation. For the 3,247 agglomerations with more than 10,000 inhabitants which are in sensitive areas and where Member States should introduce more stringent treatment, only Denmark and Austria are close to compliance with the 1998 deadline. In addition, even if a considerable number of the 542 large European cities of more than 150,000 inhabitants are equipped with adequate treatment, 37  are discharging all their wastewater into the environment without prior treatment, 72 discharge a large proportion of their wastewater without treatment or after inadequate treatment, while for 131 others no complete information has been given to the Commission.

- Identification of sensitive areas. Member States have proceeded in a restrictive fashion when designating sensitive areas and have not taken into account the fact that discharged wastewater migrates and contributes to an increase in the level of pollution of downstream water. The choice of the treatment level for a town should take into account the degree of sensitivity of the receiving water bodies situated further (and sometimes much further) downstream from the towns concerned. The incomplete designation of sensitive areas results in an under-assessment of the wastewater treatment targets for many agglomerations, which are sometimes very large, e.g. London, Paris, Athens and Dublin.

Member States slow in providing information

During 1999 and 2000 - on the basis of the information transmitted by the Member States - Commission staff has checked the degree of compliance with the first requirements of the Directive. Information from the European Environment Agency confirms that where efforts have been made to implement the Directive the result is a significant improvement in the water quality of many European rivers and lakes. Nevertheless, the situation remains worrying with regard to the eutrophication of the seawater of the North Sea, the Baltic Sea and the Adriatic Sea.

However, the Commission's work has been slowed down, in some case substantially, by the delays with which almost all Member States have provided the information. In addition, France and Germany have not given any information at all about the situation with regard to the treatment of their urban wastewater as at 31 December 1998.

Commissioner Wallström said: “This is an unconstructive attitude which is a barrier to transparency and proper information for the public.”

This situation of the individual states is as follows:

France

The decision against France concerns several failures relating to sensitive areas. These include the identification of areas in 1999 that should have been identified by the end of 1993, France’s overly restrictive approach to fixing criteria for identifying sensitive areas, and the failure to ensure - by the end of 1998 - necessary standards governing discharging into sensitive areas.

Germany

The procedure against Germany relates to remaining inadequacies in German legislation to give effect to the Directive. Legislation in place in Saxony-Anhalt sets 1 January 2004 as the deadline for more stringent treatment concerning discharges into sensitive areas. The correct deadline should have been 31 December 1998.  In addition, several pieces of German legislation (in particular for Mecklenburg-Vorpommern and Northrhine-Westfalia) fail to correctly implement the monitoring requirements of the Directive. The decision taken is to send a supplementary Reasoned Opinion, to take account of developments since an earlier Reasoned Opinion was sent on 28 October 1999.

United Kingdom

Action is being taken against the United Kingdom because of ongoing failures to identify sensitive areas, in particular for waters that are eutrophic or in danger of eutrophication. Furthermore, difficulties have been experienced with regard to the review of sensitive and less sensitive areas in Northern Ireland. It was decided that the Commission and the UK authorities will take contact to discuss these outstanding points.

New and traditional tools to improve implementation

To achieve better implementation the Commission will continue to check and help ensure compliance. It will also continue to apply traditional measures such as infringement proceedings for any failure to comply with the requirements of the Directive.

The Commission is also encouraging Member States to implement the Directive by granting financial aid under the Structural and Cohesion Funds. It is also active in the accession candidate countries via the ISPA (Financial Instrument for Structural Policies for Pre-Accession). Furthermore, it will grant financial aid conditional on stringent compliance with the requirements in the Directive.

In addition, in the next few years the Commission will step up these activities with regard to local and regional decision makers, authorities and bodies by raising awareness among the general public, activating networks of cities and by means of technical assistance to help small and medium-sized agglomerations comply by 2005.

Commissioner Wallström has also announced that she will launch a transparent and comprehensive discussion on urban water management. This discussion will address in particular preventive action at source by raising the awareness of water consumers, utilisation of the end products of treatment, and rainwater management. All the stakeholders - towns and cities, NGO's, water industries, consumers, and national, regional and local administrations - will be involved in this discussion.

She said: “It is important to change behaviour in order to put an end to the policy of ‘putting everything downs the drains’ which cannot be justified by the existence of a water-purification station. In addition, domestic, urban and industrial water consumers, who in recent years have learnt to sort their waste, should also learn to treat water as a precious commodity”.

Maps

Maps identifying sensitive areas, cities in conformity with the Directive and the situation of all large cities in the Member States are available on the Internet at the following address:

http://europa.eu.int/comm/environment/whatsnew.htm

Annex 1

Cities with more than 150.000 inhabitants without waste water treatment

Since France and Germany have not informed the Commission on the treatment situation in their large cities, the Commission could not undertake an analysis of the situation.

| Belgium | Brussels | The City of Brussels did not have any waste water treatment at all on 31 December 1998. The first treatment plant to be constructed has been treating one third of waste water at secondary level since the autumn 2000, i.e. below the level required by the Directive. The second treatment plant in Brussels is at the planning stage and will not be operational before 2004-2005. |
| --- | --- | --- |
| United Kingdom | Dundee | Projects of work are in progress in certain of those cities. |
|  | Sunderland/Whitburn |  |
|  | Middelsborouhg |  |
|  | Hull |  |
|  | Bedington |  |
|  | Torbay |  |
|  | Portsmouth |  |
|  | Brigthon |  |
|  | Hastings |  |
|  | Dover/Folkstone |  |

Annex I1

Cities with more than 10.000 inhabitants affected by a sensitive area and organic load

| Agglomerations of more than 10 000 p.e. affected by a sensitive area (SA) and organic loads - Situation on 31/12/1998 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Member States | AGGLOMERATIONS CONCERNED |  |  |  |  |  |  | IN CONFORMITY |  |  |  |  |  |  | NOT IN CONFORMITY |  |  |  |  |
|  | Number | SA load |  | Total load |  |  |  | Number |  |  | Load |  |  |  | Number |  |  | Load |  |
|  | of agglom. | (p.e.) |  | (p.e.) |  | % 1 |  | of agglom. | % |  | (p.e.) |  | % |  | of agglom. | % |  | (p.e.) | % |
| BELGIUM B | 189 | 7 801 350 |  | 9 164 000 |  | 85.1 % |  | 12 | 6.3% |  | 468 081 |  | 6.0% |  | 177 | 93.7% |  | 7 333 268 | 94.0% |
| DENMARK DK | 125 | 6 876 605 |  | 8 393 000 |  | 81.9 % |  | 123 | 98.4% |  | 6 848 167 |  | 99.6% |  | 2 | 1.6% |  | 28 439 | 0.4% |
| GERMANY D | 1685 | 109 831 358 |  | 141 458 400 |  | 77.6 % |  |  |  |  |  |  |  |  |  |  |  |  |  |
| FRANCE F | 267 | 17 868 530 |  | 70 510 000 |  | 25.3 % |  |  |  |  |  |  |  |  |  |  |  |  |  |
| IRELAND IRL | 11 | 237 000 |  | 3 918 000 |  | 6.0 % |  | 7 | 63.6% |  | 135 000 |  | 57.0% |  | 4 | 36.4% |  | 102 000 | 43.0% |
| LUXEMBOURG L | 11 | 764 500 |  | 914 000 |  | 83.6 % |  | - | - |  | - |  | - |  | - | - |  | - | - |
| NETHERLANDS NL | 263 | 15 473 498 |  | 17 218 000 |  | 89.9 % |  | - | - |  | - |  | - |  | - | - |  | - | - |
| FINLAND FI | 85 | 4 352 317 |  | 4 550 000 |  | 95.7 % |  | 11 | 12.9% |  | 478 360 |  | 11.0% |  | 74 | 87.1% |  | 3 873 957 | 89.0% |
| SWEDEN S | 144 | 7 263 240 |  | 7 496 000 |  | 96.9 % |  | 34 | 23.6% |  | 2 451 910 |  | 33.8% |  | 110 | 76.4% |  | 4 811 330 | 66.2% |
| UNITED KINGDOM UK | 212 | 13 386 805 |  | 76 528 000 |  | 17.6 % |  | 19 | 9.0% |  | 1 536 902 |  | 11.5% |  | 150 | 70.7% |  | 10 180 629 | 76.0% |

(1)  Percentage in relation to the total organic load of the Member State

(2)  First version

(3)  Second version

Nitrates Directive

Text will be drafted later.

France: France is asked to study and comment the proposed text from UK.

Norway: Norway agrees on the proposed text from UK.

UK: Text proposal in response to Article 31.i of Esbjerg Declaration:

Action under this article has effectively been taken up in the ongoing work within OSPAR to assess the eutrophication status of the various parts OSPAR maritime area through application of the Comprehensive Procedure. This is reported separately under Section B of the Nutrients section of the [5NSC] Progress Report.

2.	Agricultural policies

The Ministers therefore INVITE the European Commission, Norway and Switzerland to examine their agricultural policies, and to this end:

- i.	to consider how agricultural policies and policies for rural development, including market policies and aid policies, can contribute to the objective of reducing losses of nutrients; and

ii.	to ensure that these policies are adjusted and implemented to reach this objective. [36 i and ii]

European Commission:

Nutrients

Agri-environment

The European Commission recently prepared for a committee of Member States and Parliament a report on the state of application of Regulation (EEC) 2078/92 on agricultural production methods compatible with the requirements of the protection of the environment and the maintenance of the countryside (Council Regulation EEC/2078/92 has recently been repealed by Coucil Regulation EC/1257/1999 on support for rural development, containing the provisions for the current programming period, on which no evaluation data are yet available). As this report is a working document not in the public domain, this analysis is an abstract of the essential elements pertaining to reduction in nutrient inputs.The section of the report on input reduction, which inter alia seeks to minimize use of inputs and to promote organic farming covers a series of programmes, addresses reduced use of fertilizer and plant protection products. While three of these Member States are not OSPAR contracting parties, their experience could be instructive.

The results described are of various national programmes to implement the Regulation, which provides for premium payments to encourage environmentally friendly farming to cover income foregone, costs incurred and an incentive to provide environmental services.

So far the Regulation has been applied to 20 % of all agricultural land cover in the EU. There is nevertheless much variation as it includes more than 50 % of the arable land in L, A, FIN and S, more than 25 % in D, 10-25 % in F, IRL, I, P, and UK and less than 5 % in B, DK, EL, E sand NL. As the application of the programmes is not yet widespread in many Member States, the results indicate their potentiality rather than substantive achievement. The lack of uptake is often cited to be due to low level of payments.

In Italy, integrated production is applied to perennial crops such as vines, fruits and olives and to annual crops of various grains. Fertilizer inputs are estimated to have been reduced by 30-48 % respectively by participating farmers in the region Emiglia Romagna.

In Sachsen, Germany, it was found that a 20 % decrease in N input lead to yield decreases of 17 % and 20 % in 1995 and 1996 for winter crops (wheat, barley, rye, rape and silage wheat). For grassland a 33 % decrease from 150 kg to 100 kg N/ha in N-input lead to a 14 % yield decrease. Overall the authorities estimate a reduction of residual nitrate in soil to 116 kg N/ha.

In Denmark, a 69 % reduction was recorded by the participating farmers which given the low coverage of the programme meant very little (positive) environmental impact overall. Interestingly while the purpose of the programme was to reduce nitrate leaching, the major benefit was in enhancing biodiversity.

In Finland, with the national programme covering 90 % of cultivated area, the potential nutrient reduction of water courses is considered to have reached its objective with 30 % less P input and 10 % less N-input as well as a cessation of winter spreading of manure. Reduced livestock density has also lead to reduced leaching of nitrates.

In the UK, fertilizer use fell by 10-40 kg N/ha on average where target values of 120 kg N/ha of inorganic nitrogen and 175 kg N/ha manure were established.

As regards the significance of actual fertilizer reduction, the environmental benefit will vary depending on the type of soil and leaching therefrom as well as uptake by the crops such that the quantified change in use gives a somewhat uncertain indication of the likely change in stress on the environment.

It is premature to comment on the potentiality of programmes on extensification of livestock given the lower level of implementation.

Conclusion

This regulation is likely to make a greater contribution to nutrient reduction from agriculture if it were to be applied broadly in all Member States. Nevertheless, it is clear that substantive measures to reduce eutrophication must be taken in the framework of the nitrates and urban waste water directives which implementation requires considerable recourse to infringement proceedings.

Norway:

- The Report No. 64 to the Stortinget (1991-92) concerning Norway's implementation of the North Sea Declarations gave a broad and comprehensive assessment of measures and instruments in the Agricultural sector. Integration of environmental concerns in the Agricultural policy have been re-examined two times during the last decade. The report No. 8 Agriculture in development (1992-93) to the Stortinget established comprehensive environmental principles in the agricultural policy for sustainable development. And furthermore, The North Sea Declarations were considered as a basic challenge for reduction of  waterpollution. The report to the Stortinget No. 19 (1999-2000) concerning Norwegian Agriculture and food production gave basis for more ecological oriented agricultural production and also a basis for introducing extensive environmental planning system on the farm level. Combined use of legal and economic instruments supported with information and control has been an important strategy. In addition, to get better knowledge of the effectiveness of measures institutions are engaged for research and development of various measures and more environmental sound practices. Monitoring of the discharges of nutrients from agricultural catchments has been run over a period of 15 years.

- A Result Control system is developed and run on yearly basis to estimate and evaluate the effects of implemented measures. The system consists of three steps:

- Data collection and collocation are operated by Statistics of Norway.

- Modelling of discharge of nutrients from agriculture is carried out of Norwegian Centre for Soil and Environmental Research and The Norwegian Institute for Water Research.

- The evaluation of results and further administrative decisions and dimensions of measures necessary to achieve politically given coals are a part of the negotiations between the Government and the Farmers Unions in the Agreement of Agriculture. Thus assessment and adjustment of the policy and the use of instruments are done each year.

Switzerland:

Progress on the implementation of Paragraph 36 i, ii ED

In the revised Federal Law on Agriculture (Agricultural Policy 2002) ecological aspects in general and also the objectives to reduce nutrient inputs into waters are fully considered, e.g.:

- Reduced prize support to farmers. Prize support will decrease within 5 years (2003) of about 33%;

- For to get any direct payment, farmers have to fulfil requirements concerning fertilising, soil protection, crop rotation, crop protection and extensification (1999);

- Special programme of measures for catchments with high nitrate content in ground water and in surface waters (1999).

The result of the Programme Agricultural Policy 2002 is examined this year (2001). Of the basis of this examination, the Federal Council should decide the next programme for a perod of for years.

3.	Balanced fertilisation

The Ministers RECALL that many North Sea Member States agreed at IMM93 to aim to achieve balanced fertilisation for nitrogen and phosphorus in agricultural production by the year 2000 or 2002 at the latest, subject to an agreement on definitions.

The Ministers NOTE the progress in OSPAR in establishing an operational definition of balanced fertilisation. They ENCOURAGE OSPAR to proceed urgently with its work to establish such an operational definition. They NOTE that in many geographical areas there is a tension between meeting the nutrient needs of crops and the need to limit soil enrichment and losses of nutrients to the aquatic and atmospheric environments to environmentally acceptable levels. They EMPHASISE that the operational definition should resolve this tension at the appropriate geographical level. They further EMPHASISE that the protection of the environment should be given high priority. They finally AGREE that the concept of balanced fertilisation should take account of the principles in the further strategy to combat and prevent eutrophication in the North Sea.[37]

OSPAR:

Section C, issues not covered in sections A and B:

As far as I can see, there are only two issues to be addressed in the NSC Progress Report not covered in the sections A and B: balanced fertilisation and ammonia emissions.

Balanced fertilisation (cf. ED §37):

In 1996, OSPAR endorsed an agreement reached at PRAM 1996 that:

a.	as a starting point, the following definition should be considered for further discussion on the definition of balanced fertilisation: "Balanced Fertilisation" means the supply of nutrients, from any source:

(i)	which does not exceed the requirement of the crops; and

(ii)	where the resulting soil enrichment and losses of nutrients to the aquatic and atmospheric environment are at environmentally acceptable levels;

b.	Contracting Parties, which could not accept this definition, submit explanations of their reservations, together with constructive proposals on how these reservations could be lifted, to NUT 1996. (Cf. OSPAR 96/17/1, §3.68)

At NUT 1996, it became apparent that there was no scope for solving the different views and opinions of Contracting Parties in order to establish a commonly agreed definition of balanced fertilisation. NUT therefore agreed that:

a.	this conclusion should be reported to HOD November 1996 with a recommendation to delete topic 14 ("to adopt a Decision/Recommendation on balanced fertilisation") from the programme of work for MMC 1997;

b.	Contracting Parties should submit to NUT 1997 proposals which might help NUT to find a new start in the discussion on a BEP for agriculture (e.g. on issues and measures, which would form part of such a BEP, or on how to address the concept of "balanced fertilisation" in a larger context). (Cf. NUT 96/15/1, §6.5)

HOD November 1996 noted the outcome of NUT 1996 and agreed that "there was at present no scope for finding a commonly agreed definition of balanced fertilisation and, consequently, no way of preparing an OSPAR measure on this issue. Nevertheless, MMC 1997 should be informed about OSPAR's past and present work with regard to balanced fertilisation, together with an outline for the future approach to be taken." To this end, the Chairman of OSPAR undertook to present a document for consideration at HOD May 1997, which would include a text proposal for inclusion in the Ministerial Declaration from MMC 1997 (cf. HOD(2) 96/13/1, §§5.7-5.8).

After further discussion, the Section on Eutrophication of the Sintra Statement contains a simple reference to balanced fertilisation as follows:

"as one of the main elements of the source-oriented actions promote good housekeeping in industry and sewage treatment and good agricultural practice, ecological agriculture and balanced fertilisation;"

The issue of balanced fertilisation is addressed in §3.5.a(ii) of the OSPAR Strategy to Combat Eutrophication as a basic requirement of the 'source oriented approach':

"the promotion of good housekeeping in industry and sewage treatment and of good agricultural practice and ecological agriculture including proper use of the approach of aiming to strike a balance between the amounts of nutrients in the fertilizer applied and the requirements of the crop, and that proper attention is given to ammonia emissions;"

In the context of discussions regarding measures with respect to agriculture, further discussion took place at NUT 1997 cf. NUT 97/13/1, §§4.6-4.11), NEUT 1998 (cf. NEUT 98/98/15/1, §§8.6-8.11), NEUT 1999 (cf. NEUT 99/15/1, §§7.4-7.7) and EUC 2000 (cf. EUC 00/15/1, §§8.1-8.5).

EUC 2000 agreed that it should consider the need for possible additional measures with respect to agriculture in the light of the outcome of the examination of:

- a consideration of the need for further work on PARCOM Recommendation 92/7 (planned for the 2002/2003 intersessional period);

b.	the reporting on the implementation of PARCOM Recommendation 92/7 (planned for the 2001/2002 intersessional period); (cf. EUC 00/15/1, §§8.4-8.5).

4.	Atmospheric deposition and ammonia emissions

The Ministers furthermore AFFIRM that they consider atmospheric deposition to contribute substantially to the anthropogenic input of nitrogen to the North Sea. The Ministers EMPHASIZE the agreement already stated at the IMM93 Statement of Conclusions concerning atmospheric deposition and:

i.	INVITE the European Commission, Norway and Switzerland to continue to progress the work on limiting atmospheric emissions of nitrogen oxides including emissions from the transportation sector;

ii.	INVITE the Commission to the UN-ECE Convention on Long-range Transboundary Air Pollution to pursue its work on reductions of emissions of nitrogen oxides, especially with regard to the North Sea; and

iii.	NOTE that OSPAR in its Progress Report<8> to the 4NSC has reported that ammonia emissions represent 30 to 50 per cent of the total emission of nitrogen to the atmosphere. In order therefore to meet the need for an overall reduction of ammonia emissions as seen by some North Sea States, the Ministers therefore INVITE OSPAR in cooperation with other appropriate bodies to consider the justification for further reduction targets for the different sources of ammonia emissions e.g. livestock production, and in the light of a justification to consider BEPs for the reduction of these emissions.[38]

European Commisson:

Atmospheric deposition not covered in the submitted EU report.

Norway:

Limiting atmospheric emissions of nitrogen oxides from sources in Norway

- NOX emissions in Norway, sources and major trends

The main sources of NOx emissions in Norway are coastal shipping and fisheries with 40 % of the emissions, road traffic 25% and  the oil/gas industry offshore with almost 20 % of the emissions in 1999.

The overall NOx emissions decreased in the period 1987 – 1992 from 229.000 to 208.000 tonnes due to reduced economic activity and the introduction of tighter emission standards for vehicles. From 1992 to 1999 emissions increased to 230.000 tonnes, due to increased activity in the transportation sector, especially navigation and aviation and increased flaring offshore. This was about 1000 tonnes over the figures for 1987, and the obligation according to the Sophia Protocol.

The projection towards 2010 shows a decrease, a.o because of already approved tighter emission standards for vehicles.

- International agreements

Norway was one of the countries which signed the Sophia protocol in 1988 with the commitment to stabilize NOx emissions at 1987 levels by 1994. Twelve countries, a.o Norway went beyond this and undertook to reduce the emissions by 30 percent from 1986 levels by 1998. Norwegian NOx emissions were consistently below the 1987 levels until as recently as 1998 only to exceed them for the first time in 1999. The target of 30 per cent reductions by 1998 was not met.

The Gothenburg Protocol implies that Norway has undertaken to reduce its emissions to

156 000 tonnes nitrogen oxides (NOx) in 2010. This corresponds to a reduction of 28 per cent compared with the 1990 level, or 74.000 tonnes below the level in 1999.

- Instruments

To limit the emissions of NOx a number of instruments has been used. The most important has been or are;

- Implementation of tighter emission standards for road traffic
1985: ECE- standard (R15-04) 
1989: US 87 standard for light vehicles
1993: EURO 1 standard for heavy duty vehicles
1996: EURO II  ”                   ”                   ”
1997: EU standard for light vehicles
2000: Auto-Oil I standard

- Implementation of  88/609/ECE standards for big stationary  combustion plants.

- NOx reduction program for vessels in coastal navigation.ing financial and technical support to improvements. The program 1996-2000 gave financial support and technical assistance to measures for reduction of NOx emissions from vessels in coastal navigation and resulted in a reduction of 1400 tonnes of NOx/year.

- Implementation of IMO (International Maritime Organisation) emission standards for new engines in ships; Technical Code on Control of Emission of Nitrogen Oxides from Marine Diesel Engines

- Funding of measures to lower the emissions from domestic ferries

- A cross sectorial analysis of possible measures to reduce the emissions of NOx was carried out in 1999.  The analysis comprised both cost and effect of measures and showed that a reduction of about 30 % compared with the 1990 level can be obtained in 2010 with reasonable costs. Reductions up to 75 % are technical possible. This analysis will be useful in the further process to fulfil the commitments of the Gothenburg Protocol. However, the realization of these measures is dependent on introduction of new instruments.

Reduction of NOx Emissions in Norway – Analysis of Abatement Measures for the End-year 2010, SFT report 99:13, ISBN: 82-7655-175-0

Other policies and instruments aiming at targets like reductions of emissions of green house gases, noise and improvements of local air quality can in many cases also lead to lower NOx emissions.

Examples are;

Tigtening of criteria/standards for air quality, comprising a.o NOx;

- 1992 SFT’s criteria for air quality from 1992; 100 ug/m3,

- 1997 limits triggering of the need for monitoring (200 ug/m3) or action (300 ug/m3

- 1998 National targets and standards for air quality; 150 ug/m3 which is under EU’s suggested standards of 200 ug/m3

All the instruments above are part of already approved policy and are therefor together deciding for the development of the NOx emissions as described above (base line)

Additional instruments will be necessary to realize the measures for a further reduction of the national emissions for compliance with the Gothenburg Protocol.

OSPAR:

Ammonia emissions (cf. ED §38iii)

At NUT 1996, the comment was made that "this overview (cf. NUT 96/9/1) indicated that ammonia from agriculture contributed to the atmospheric emissions, and the resulting deposition, of nutrients. Taking into account that the draft OSPAR measure on the Reduction of Nutrient Inputs from Agriculture, which should be presented to NUT 1997, should address ammonia emissions (cf. § 6.9 d (v)) and that considerable amount of work and action concerning ammonia emissions was being carried out by other international organisations (e.g. CLRTAP, EMEP), there was currently no justification or need for further work on reduction measures to be initiated within the framework of OSPAR. However, the re-assessment of the need and scope for additional reduction measures on ammonia emissions should be a part of the effectiveness assessment of such an OSPAR measure on the Reduction of Nutrient Inputs from Agriculture" (cf. NUT 96/15/1, §9.11.b)

At NUT 97 some further discussion took place but no concrete action was agreed upon (cf. NUT 97/13/1, §§8.13-8.14).

Further discussion took place in the context of a discussion regarding measures with respect to agriculture (cf. §§10-11 above). The reduction of ammonia emissions is also mentioned in the OSPAR Strategy to Combat Eutrophication (see also §9 above).

Switzerland:

# Progress on the implementation of Paragraph 38 i, ii ED

In 2001 a mileage, weight and emission-dependent tax for heavy duty vehicles was introduced. Switzerland also adopted the tight exhaust standards of the EU for light duty vehicles (EURO 3 as from 2000, EURO 4 as from 2005) and for heavy duty vehicles (EURO 3 as from 2000, EURO 4 as from 2005, EURO 5 as from 2008). They will lower atmospheric emissions of nitrogen oxides substantially. An incentive tax on non-renewable energy and an energy fee to promote renewable energy use were rejected in a public vote in 2000. Large infrastructure projects to enhance capacity and attractiveness of public transport and to transfer freight traffic from the road to the rail are being carried out.

UN-ECE:

Progress report on work done to pursue reductions of emission of nitrogen oxides within the framework of the Convention on Long-range Transboundary Air Pollution to the fifth North Sea Conference on the Ministerial Declaration on the Protection of the North Sea.

I 	The 1988 NOx Protocol

Emissions of nitrogen oxides are covered by the 1988 Sofia Protocol to the 1979 Convention on Long-range Transboundary Air Pollution on the Control of Emissions of Nitrogen Oxides or their Transboundary Fluxes.  This Sofia Protocol had, as of 15 March 2001, 28 Parties.  Its main obligation requires Parties to stabilize by 1994 their NOx emissions at 1987 levels, but Parties may choose an alternative base year.  The Implementation Committee established under the Convention to review compliance by Parties with their Protocol obligations reviewed the implementation of this obligation in 2000 and reported the following results to the Executive Body for the Convention:

“The emission reduction obligation for the Parties to the NOx Protocol (paragraph 1 of article 2) is to control and/or reduce their total annual emissions of nitrogen oxides or their transboundary fluxes so that these, at the latest by 31 December 1994, do not exceed such emissions for 1987.  At its fourteenth session in December 1996, the Executive Body confirmed its understanding that the obligation "should be taken to mean that emission levels for the years after 1994 should not exceed those specified in that paragraph."

Official submissions suggest that this requirement has not been met in several of the years 1994 - 1998 by 3 of the [in 2000] 27 Parties to the Protocol.  The following Parties are in non-compliance with the obligation of paragraph 1 of article 2 of the Protocol:

- 	Ireland ratified the Protocol in 1994.  Its emissions were 4 % above the 1987 level in 1996, 3 % in 1997 and 6 % in 1998;

- 	Spain’s emissions were 21 % above the 1987 level in 1994 and 1995 and 19 % in 1996.  No data has yet been reported for subsequent years;

- 	The United States of America specified a different base year (1978) when it signed the Protocol, with the consequence that its obligation became to control and/or reduce its total annual emissions of nitrogen oxides or their transboundary fluxes so that these, at the latest by 31 December 1994, did not exceed 1978 level (21,830 kt).  In addition, it had to ensure that its national average annual transboundary fluxes or national average annual emissions for the period from 1 January 1987 to 1 January 1996 did not exceed those for the calendar year 1987. Emission data provided by the United States of America shows that it did not achieve stabilization of emissions at 1978 levels in 1994. Its emissions were 5 % above the 1978 level in 1994, 4 % in 1995, 2 % in 1996, 3 % in 1997 and 1 % in 1998.  Additionally, the average of its annual emissions in the period 1 January 1987 to 1 January 1996 (22,128 kt) was above its emissions in 1987 (20,689 kt).

Under paragraph 1 (b) of article 8 of the Protocol, Parties are required to report annually their levels of national emissions of nitrogen oxides. Since the following Parties have not reported emission data for the base year, it has not been possible to assess their compliance for any year:

- 	Greece has not provided emissions data for the base year;

- 	Luxembourg has submitted no emission data for the base year (1987), nor for the years 1991, 1992, and 1993;

- 	The Russian Federation has reported partial data for the base year (1987), not covering all sources.  (The absence of complete data may affect the Russian Federation’s compliance with its emission-related obligations.);

-	The European Community has not reported NOx emissions for any year.

The following Parties have not reported emission data for the most recent year (1998):

- 	Finland, France, Hungary and Slovakia have submitted only preliminary data;

- Italy, Liechtenstein, Spain and Ukraine have not reported any data.”

At the meeting of the Executive Body and with reference to the above text, the delegation of the United States indicated that it had experienced national implementation problems with the diesel engine manufacturers.  This had required a reassessment of NOx emission estimates.  New emission estimates had been reported for recent years, but not yet for the base year of the Sofia Protocol.  With updated emission estimates also for the base year, the United States would still have to be considered to have been in non-compliance in 1994 by 1.2 to 3 %, but was back in compliance since.  The delegation would submit more detailed information to the secretariat.

II	1999 Protocol to Abate Acidification, Eutrophication and Ground-level Ozone

As a second step to the Sofia Protocol, the Executive Body for the Convention conducted negotiations on further reductions of nitrogen oxides to control the environmental effects related to this pollutant.  As a result of these deliberations and based on elaborate scientific and technical work, it adopted in 1999 the Gothenburg Protocol to the 1979 Convention on Long-range Transboundary Air Pollution to Abate Acidification, Eutrophication and Ground-level Ozone.  31 Parties to the Convention signed this Protocol.

The Protocol deals with the environmental and health effects related to NOx and consequently covers obligations related also to other air pollutants responsible for these effects.  It sets emission ceilings for 2010 for four pollutants: sulphur, NOx, VOCs and ammonia.  These ceilings were negotiated on the basis of scientific assessments of pollution effects and abatement options.  Countries whose emissions have a more severe environmental or health impact and whose emissions are relatively cheap to reduce will have to make the biggest cuts. Once the Protocol is fully implemented, Europe’s sulphur emissions should be cut by at least 63%, its NOx emissions by 41%, its VOC emissions by 40% and its ammonia emissions by 17% compared to 1990.

The Protocol also sets tight limit values for specific emission sources (e.g. combustion plant, electricity production, dry cleaning, cars and lorries) and requires best available techniques to be used to keep emissions down. VOC emissions from such products as paints or aerosols will also have to be cut. Finally, farmers will have to take specific measures to control ammonia emissions.  Guidance documents adopted together with the Protocol provide a wide range of abatement techniques and economic instruments for the reduction of emissions in the relevant sectors, including transport.

It has been estimated that once the Protocol is implemented, the area in Europe with excessive levels of acidification will shrink from 93 million hectares in 1990 to 15 million hectares. That with excessive levels of eutrophication will fall from 165 million hectares in 1990 to 108 million hectares. The number of days with excessive ozone levels will be halved. Consequently, it is estimated that life-years lost as a result of the chronic effects of ozone exposure will be about 2,300,000 lower in 2010 than in 1990, and there will be approximately 47,500 fewer premature deaths resulting from ozone and particulate matter in the air. The exposure of vegetation to excessive ozone levels will be 44 % down on 1990.

III	Development of overall nitrogen oxides emissions

Based on emission data officially submitted by June 2000 (interpolating for missing data), the picture shown in the figure below emerges.  The emissions of NOx are characterized by relatively high releases in the late 1980s and an easing-off in the 1990s.  The reduction is 15% between 1980 and 1998.

Figure. Emissions of nitrogen oxides in the EMEP area 1980-2010  (millions of tonnes as NO2)

Compilation of Submitted Inputs

Further reduction of nutrient inputs to the North Sea

Part II:

Inputs of nutrients using HARP-NUT Guidelines

Data on nutrient inputs submitted from France

figures (tonnes/year)

Country	Specific 	Riverine load,	Riverine load,	Riverine load,	Riverine load,	Transboundary	Transboundary

Year	annual 	without	without	flow- 	flow-	riverine 	riverine

Catchment 	riverine flow	normalisation	normalisation	normalised	normalised	inputs, 	inputs,

mill m3/year	 tonnes/yr 	 tonnes/yr 	tonnes/yr 	tonnes/yr 	tonnes/yr 	tonnes/yr

France

Data from	1990

Basse-Normandie                                   	772	278,7241397462	6011,205190562	459,6655270855	9913,543207469	0	0

Bretagne Nord                                     	1248	215,0340236470	7295,759466888	300,1044144146	10182,06135655	0	0

Cotentin                                          	531	84,32186881731	224,5258543929	197,6604272627	526,3151414134	0	0

Côtiers haut-normands                             	201	52,33188179609	1122,805519299	62,71428029017	1345,564837951	0	0

Côtiers picards et boulonnais                     	1748	368,9205363364	8622,500861624	30,74337802803	718,5417384687	0	0

Seine à l'aval de Paris et Risle                  	10527	10853,66414619	83698,91197857	15527,77339127	119743,6847863	0	0

Versants mer du Nord et transfrontaliers     	584	306,0958970479	6478,689586129	25,50799142066	539,8907988441	0	0

Sum	1990	15611	12159,09249358	113454,3984574	16604,16940977	142969,6018670	0	0

Data from	1991

Basse-Normandie                                   	634	188,2973006675	4518,914586406	378,0039660906	9071,652275580	0	0

Bretagne Nord                                     	1210	187,4881101389	7608,336833235	269,9598659617	10955,07117848	0	0

Cotentin                                          	506	91,16305303599	198,6283844642	224,4272747693	488,9878687975	0	0

Côtiers haut-normands                             	207	67,68061781129	1151,047680228	78,95335296448	1342,763655459	0	0

Côtiers picards et boulonnais                     	1723	482,3577127152	7984,122068279	40,19647605960	665,3435056899	0	0

Seine à l'aval de Paris et Risle                  	11466	9754,578353745	93698,35843666	12812,15650399	123068,1623462	0	0

17. august 2001	Page 1 of 5

Country	Specific 	Riverine load,	Riverine load,	Riverine load,	Riverine load,	Transboundary	Transboundary

Year	annual 	without	without	 flow-	flow-	riverine 	riverine

Catchment 	riverine flow	normalisation	normalisation	normalised	normalised	inputs, 	inputs,

mill m3/year	 tonnes/yr 	 tonnes/yr 	tonnes/yr 	tonnes/yr 	tonnes/yr 	tonnes/yr

Versants mer du Nord et transfrontaliers     	771	542,5087175502	7347,505629520	45,20905979585	612,2921357933	0	0

Sum	1991	16517	11314,07386566	122506,9136188	13848,90649963	146204,2729660	0	0

Data from	1992

Basse-Normandie                                   	1238	445,5473010954	9943,849259198	458,0560760159	10223,02359584	0	0

Bretagne Nord                                     	1200	266,3347787200	9435,126611418	386,6446034487	13697,20021055	0	0

Cotentin                                          	750	130,5351170458	285,4566588824	216,9073901623	474,3371767222	0	0

Côtiers haut-normands                             	182	49,64696112704	1050,153769818	65,71432105425	1390,017443557	0	0

Côtiers picards et boulonnais                     	1875	600,1301271447	10006,70090760	50,01084392873	833,8917423000	0	0

Seine à l'aval de Paris et Risle                  	11394	8973,954495448	4419,905926618	11860,97278958	5841,837503714	0	0

Versants mer du Nord et transfrontaliers     	873	546,9668018026	7567,715397724	45,58056681688	630,6429498103	0	0

Sum	1992	17512	11013,11558238	42708,90853126	13083,88659101	33090,95062249	0	0

Data from	1993

Basse-Normandie                                   	1450	433,1996039460	12215,54973491	380,1691980257	10720,17541997	0	0

Bretagne Nord                                     	1837	422,9839246144	9906,218847879	400,9865802278	9391,044405339	0	0

Cotentin                                          	1445	353,9207885518	4622,553144990	305,0628632321	3984,420648539	0	0

Côtiers haut-normands                             	293	63,98299011805	1640,416384250	52,74710191589	1352,347085451	0	0

Côtiers picards et boulonnais                     	2198	521,7627902140	9026,743098138	43,48023251783	752,2285915115	0	0

Seine à l'aval de Paris et Risle                  	13336	10749,18316382	144027,4465861	12138,88304724	162647,9243172	0	0

Versants mer du Nord et transfrontaliers     	1158	516,9419072354	10173,75424808	43,07849226962	847,8128540067	0	0

Sum	1993	21717	13061,97516850	191612,6820443	13364,40751543	189695,9533220	0	0

17. august 2001	Page 2 of 5

Country	Specific 	Riverine load,	Riverine load,	Riverine load,	Riverine load,	Transboundar	Transboundar

Year	annual 			 		y riverine 	y riverine

Catchment 	riverine flow	without 	without 	flow-	flow-	inputs, 	inputs,

normalisation	normalisation	normalised 	normalised

mill m3/year	 tonnes/yr 	 tonnes/yr 	tonnes/yr 	tonnes/yr 	tonnes/yr 	tonnes/yr

Data from	1994

Basse-Normandie                                   	2156	512,2263403324	16294,74786589	302,4348393855	9620,941106717	0	0

Bretagne Nord                                     	2708	479,4552364313	13175,68395523	308,3837175133	8474,547966432	0	0

Cotentin                                          	2433	447,7769798109	6150,269079570	229,3316400731	3149,896842630	0	0

Côtiers haut-normands                             	409	75,87475215214	2392,408637484	44,72913005228	1410,355276939	0	0

Côtiers picards et boulonnais                     	3224	717,4466668987	13027,89871957	59,78722224156	1085,658226630	0	0

Seine à l'aval de Paris et Risle                  	20614	10970,17977946	159299,0432721	8014,504813180	116379,4007669	0	0

Versants mer du Nord et transfrontaliers     	1405	603,7707821815	12292,24355886	50,31423184846	1024,353629905	0	0

Sum	1994	32949	13806,73053727	222632,2950887	9009,485594295	141145,1538161	0	0

Data from	1995

Basse-Normandie                                   	2097	538,6347164410	15566,31872322	326,9930151446	9449,961799049	0	0

Bretagne Nord                                     	2777	521,5985193544	12678,91945799	327,1367873844	7951,979971354	0	0

Cotentin                                          	1953	392,2027064137	3971,964486294	250,1391789150	2533,240895673	0	0

Côtiers haut-normands                             	497	110,7181526580	3086,503535903	53,76617862015	1498,846362941	0	0

Côtiers picards et boulonnais                     	3090	526,5502384417	13290,10003146	43,87918653681	1107,508335955	0	0

Seine à l'aval de Paris et Risle                  	22674	10638,68431805	164688,6490315	7066,069444422	109383,9610213	0	0

Versants mer du Nord et transfrontaliers     	929	186,3646821668	6823,421172606	15,53039018056	568,6184310505	0	0

Sum	1995	34017	12914,75333353	220105,8764390	8083,514181204	132494,1168173	0	0

Data from	1996

Basse-Normandie                                   	1033	314,4332194686	6992,245926663	387,3400054769	8613,519208011	0	0

Bretagne Nord                                     	1309	204,7987509751	10448,13278669	272,3951476887	13896,67007221	0	0

17. august 2001	Page 3 of 5

Country	Specific 	Riverine load,	Riverine load,	Riverine load,	Riverine load,	Transboundary	Transboundary

Year	annual 	without	without	flow- 	flow-	riverine 	riverine

Catchment 	riverine flow	normalisation	normalisation	normalised	normalised	inputs, 	inputs,

mill m3/year	 tonnes/yr 	 tonnes/yr 	tonnes/yr 	tonnes/yr 	tonnes/yr 	tonnes/yr

Cotentin                                          	728	203,7871099901	1513,728194817	348,5521764528	2589,041362268	0	0

Côtiers haut-normands                             	176	32,16200468250	1043,908888225	44,19988907410	1434,632496280	0	0

Côtiers picards et boulonnais                     	1533	316,1543430529	8334,334042415	26,34619525441	694,5278368679	0	0

Seine à l'aval de Paris et Risle                  	11718	8458,248727191	100194,7559019	10870,49801545	128769,7879692	0	0

Versants mer du Nord et transfrontaliers     	366	349,3650604766	2976,115349125	29,11375503972	248,0096124271	0	0

Sum	1996	16863	9878,949215837	131503,2210899	11978,44518444	156246,1885572	0	0

Data from	1997

Basse-Normandie                                   	1042	247,5526412334	7244,177281431	302,5465653703	8853,474333823	0	0

Bretagne Nord                                     	1118	185,1466343468	6388,379686976	288,4235038386	9951,889537013	0	0

Cotentin                                          	1164	264,5915198233	3115,434398052	283,3121786444	3335,860904859	0	0

Côtiers haut-normands                             	144	31,96050975608	99,56456261165	53,46189255144	166,5464659011	0	0

Côtiers picards et boulonnais                     	1433	327,6637397150	4385,176852604	27,30531164292	365,4314043837	0	0

Seine à l'aval de Paris et Risle                  	13129	9337,382026787	105501,3456579	10710,29179972	121013,5982462	0	0

Versants mer du Nord et transfrontaliers     	390	158,4665486996	1078,388568300	13,20554572496	89,86571402500	0	0

Sum	1997	18420	10552,76362036	127812,4670079	11678,54679750	143776,6666062	0	0

Data from	1998

Basse-Normandie                                   	1615	343,6462624391	12384,62845408	270,7790173034	9758,574118232	0	0

Bretagne Nord                                     	2216	377,7703411735	23007,40848050	296,9158777063	18083,11595217	0	0

Cotentin                                          	1980	402,3981405470	14776,59638680	253,2404430346	9299,326807155	0	0

Côtiers haut-normands                             	216	33,19726927241	194,9004645148	37,17903391709	218,2773203782	0	0

Côtiers picards et boulonnais                     	887	151,5956450019		12,63297041683		0	0

Seine à l'aval de Paris et Risle                  	16879	8804,342268502	5269,546706798	7855,553782642	4701,680864167	0	0

17. august 2001	Page 4 of 5

Country	Specific 	Riverine load,	Riverine load,	Riverine load,	Riverine load,	Transboundary	Transboundary

Year	annual 	without	without	 flow-	flow-	riverine 	riverine

Catchment 	riverine flow	normalisation	normalisation	normalised	normalised	inputs, 	inputs,

mill m3/year	 tonnes/yr 	 tonnes/yr 	tonnes/yr 	tonnes/yr 	tonnes/yr 	tonnes/yr

Versants mer du Nord et transfrontaliers     	790	60,8357917008	2124,224292902	5,0696493084	177,0186910752	0	0

Sum	1998	24583	10173,78571863	57757,30478561	8731,370774328	42237,99375318	0	0

Data from	1999

Basse-Normandie                                   	1280	323,0733580588	10165,05683930	321,2333061485	10107,16214823	0	0

Bretagne Nord                                     	2506	370,6911616245	23940,29677739	257,6408196202	16639,18194501	0	0

Cotentin                                          	2022	425,0884261619	14644,25419775	261,9368943520	9023,700078816	0	0

Côtiers haut-normands                             	295	61,15764624850	333,5082555849	49,96168541119	272,4538233512	0	0

Côtiers picards et boulonnais                     	1141	177,1450813550		14,76209011291		0	0

Seine à l'aval de Paris et Risle                  	18862	13726,94068480	5220,272447843	10959,86657255	4167,970913143	0	0

Versants mer du Nord et transfrontaliers     	913	28,04936104927	1600,195228481	2,337446754106	133,3496023734	0	0

Sum	1999	27019	15112,14571930	55903,58374636	11867,73881494	40343,81851093	0	0

17. august

Further reporting from France by clicking on the icon below

20

ggg

Background to the Reductions of Inputs of Nutrients reported by the UK

This paper complements the estimates of reductions in the inputs (waterborne loads via the aquatic pathway) of the nutrients nitrogen (N) and phosphorus (P) reported by the UK to the 5NSC. It provides background information on the sources of data and describes the methods used to assess reductions. The reliability and limitations of the data are considered. The information provided in this background documentation should not be used out of the context in which it has been supplied.

General Aspects

The aim is to provide best estimates of UK reductions in the inputs of N and P since 1985. This is in the context of the reports made to the 5NSC by other North Sea States that have identified marine eutrophication problem areas and are, therefore, subject to the commitment to achieve 50% reductions in inputs over the period 1985 to 1995. (The outcome of the current assessment of eutrophication status of the OSPAR maritime area - under the Common Procedure - will determine the extent to which the UK will be subject to the 50% reduction commitment).

Additionally, information is provided on inputs and reductions of N and P from the Thames catchment (ICES Zone 12). This is to update Table 3.4.5 of the Progress Report to the 4NSC and to fulfil the recommendation of OSPAR 2000 in respect of reporting nutrient inputs via major North Sea rivers (ref section 9.13 of Summary Record OSPAR 2000).

Generally, in estimating reductions in inputs, the load approach has been used, ie one of the two basic methods for reporting contained in the HARP NUT Guidelines. (The load approach accords with the method used by the UK in its reporting of baseline estimates to the 3NSC and Thames inputs to the 4NSC).

Table 1 below provides an overall summary of reductions achieved in inputs of nutrients for the UK and for the Thames catchment respectively. More detailed information relating to the inputs and emissions data and any trends is given in the following section and in Annexes 1-5.

Table 1: Reductions 1985-1999 in UK Inputs of Nutrients for ‘All UK’ and Thames Catchments

| Catchment Area | 1985 Input Nitrogen | 1985 Input Phosphorus | 1999 Input Nitrogen | 1999 Input Phosphorus | Reduction Nitrogen | Reduction Phosphorus |
| --- | --- | --- | --- | --- | --- | --- |
|  | kt/year | kt/year | kt/year | kt/year | % | % |
| All UK | 319 | 58.4 | 355 | 33.0 | -11 (A) | 43 |
| Thames | 40.5 | 10.9 | 29.0 | 6.0 | 28 | 45 |

Note (A): 1985 base line figure appears to be an underestimate, so the indicated increase will be a worst case.

The data for 1990-1999 indicate no underlying change in inputs of nitrogen.

Data and Assessment Method

Use has been made of the most recent OSPAR RID data for riverine and direct inputs to supplement the data already reported in 1990 (3NSC) and 1995 (4NSC). Only Total-N and Orthophosphate-P data is available for the 1985 baseline year. Although more detailed nutrient information became available in the 1990s, in order to be consistent over the period 1985-1999, the assessment of reductions is based on the pattern of change in the inputs of Total-N and Orthophosphate-P. Data abstracted from the relevant sources is given in Annexes 1, 2, 4 and 3.

It is recognised that there is variability in any estimates for a given year due to natural factors, such as weather. There may be other variability due to refinements in analytical techniques or gaps in reporting. Also, estimates for a given year are likely to be more reliable towards the end of the time series than at the beginning (when there was little or no basis for comparing estimates and checking for anomalies). In order to limit the effect of such variability, consideration has been given to the whole time series of data available and a smoothed curve determined for each series of nutrient inputs. More is given on the refinement of this graphical technique below.

As noted above, direct and riverine inputs of nutrients are both taken into account in UK input reduction estimates. Diffuse inputs direct to the sea, eg inputs via leaching and surface water run off downstream of the riverine monitoring point have not been allowed for. This leads to a small (order 10%) under-estimation of the true annual inputs. As this under-estimation is consistent year on year, it such it should not materially affect the estimates of reductions in inputs.

As most results for nutrients monitoring are above the level of detection (LOD), there is no significant difference in the upper and lower estimates reported to OSPAR. In view of this, only upper estimates have been used in this assessment of reductions.

Generally, it is acknowledged within OSPAR that any individual direct or riverine input estimate is only accurate to within +/- 30-50%. The aggregation of individual estimates and “smoothing” such aggregated estimates over a period will increase precision. Generally, even with such mitigating factors it would be unwise to assume that any “smoothed” estimate of input is better than +/-10% in its accuracy. It follows that any estimate of reduction has the same order of reliability.

A factor affecting riverine estimates is the variability arising due to flow variation. In the UK, the 1990s have been marked by extremes in riverine flow rate, with 1996 and 1997 being well below average and 1998 and 1999 being well above average. This introduces variability for UK riverine input estimates of the order +/- 20%. Because of flow extremes, inputs for 1996 and 1997 are likely to be under-estimates and inputs for 1998 and 1999 are likely to be over-estimates. An abstract of riverine flow data reported to OSPAR is given in Annex 1.

In order to compensate for this flow related variation, a simple flow adjustment of riverine inputs has been applied. This involves dividing the riverine input for a given year by a flow-weighting factor (produced by dividing the long term average (LTA) flow by the actual flow for the given year). This adjustment has been applied to the input data from 1991 to 1999, these being the years that flow data have been reported to OSPAR under RID. The charts given in Annexes 2, 3, 4 and 5 show how the inputs vary before and after flow adjustment.

For Total-N and Orthophosphate-P, where UK 1985 baseline input estimates have been reported to the 3NSC, a smoothed curve was drawn to reflect the flow adjusted inputs but constrained to pass through the 1985 estimate. Reduction estimates for these substances are based on the “smoothed” estimate for 1999. The smoothed curves and reduction estimates are given in Annexes 3 and 5.

Although for the more recent data sets, direct and riverine estimates were derived, UK reduction estimates are based on the total (riverine plus direct) inputs. This is because the original 1985 estimates that are available are total input estimates. Also, for assessing national reductions, there is no general benefit to be gained by considering reductions of direct and riverine inputs separately. However, it is worth noting that direct inputs of N and P from the UK have decreased by about 30% and 35% respectively over the period 1990-1999.

The above approach of considering inputs to assess UK reductions is subject to any limitations in the 1985 baseline estimates. From the consideration of UK annual input estimates throughout the 1990s, it would appear that the 1985 input of nitrogen is probably an underestimate. Consequently, the 11% increase in nitrogen indicated is very much a worst case estimate. Consideration of the more consistent inputs data over the period 1991-1999 would indicate that there has probably been no significant underlying change in overall inputs of nitrogen since 1985.

Thames

Reductions in nutrient inputs relating to the Thames catchment are given in Table 1 above. The Table in Annex 6 provides the data on inputs and flows in the same format as Table 3.4.5 of the Progress Report to the 4NSC. The additional input data, for the period 1994-1999, is that reported to OSPAR for ICES Zone 12. The chart in Annex 6 shows how flows and inputs have varied. A smoother curve has been added to indicate the underlying pattern of change in the inputs of nitrogen, which have been highly variable in recent years.

Conclusions

1. 	General

Input estimates are valuable in that they provide an annual estimate of the net aquatic input of nutrients to the marine environment from both point and diffuse land based sources. Typically for the UK, nitrogen and phosphorus inputs are of the order 350kt/year and 35kt/year respectively.  In addition to the absolute value of input, inputs data allows the year on year variation to be evaluated. In respect of riverine inputs, this variation is significant. For nitrogen, UK riverine inputs varied in the range 200-340 kt/year during the 1990s.

2. 	UK (National) Reductions

From the analysis of flow adjusted inputs of Total-N and Orthophosphate-P, UK inputs of nitrogen would appear to have increased by 11% and the inputs of phosphorus have reduced by 43%.

The 11% increase indicated above is very much a worst case. This is due to limitations in the 1985 baseline estimate. Consideration of the more consistent inputs data over the period 1991-1999 would indicate that there has been no significant underlying change in overall inputs of nitrogen since 1985.

Flow variations have lead to wide variations in the riverine inputs of nitrogen and phosphorus, especially in recent years.

Direct (sewage and industrial) inputs of both nitrogen and phosphorus have reduced by about 30% and 35% respectively over the period 1990-1999 for which the related OSPAR RID inputs data is available.

3.	Thames (ICES Zone 12) Reductions

Since 1985, there has been a reduction in inputs of both nitrogen and phosphorus from the Thames catchment. Not taking into account the wide flow related variations, actual reductions were 15% and 35% respectively. Consideration of “smoothed” inputs (which balance out the inter-annual variations), indicates that over the period 1985-1999 the reductions in inputs of nitrogen and phosphorus were of the order 28% and 45% respectively.

# Report from Belgium

Catchment description summary report – national figures

Catchment name	Catchment 	Lake 	River 	Total 	Long term

Water 	Water 	Populatio

Area	Surface 	Surface	n	annual

flow

Meuse	13824	0	0	2451043	5500

Rhine	767	0	0	33371	0

Scheldt	0	0	0	7670422	4244,8

Seine	79	0	0	2292	0

Whole country	0	0	0	0	0

Sum	Belgium	14670	0	0	10157128	9744,8

Retention in surface waters summary report – national figures (tonnes/year)

Country	Tot-N	Tot-N	Tot-N	Tot-P	Tot-P	Tot-P 	Tot-N	Tot-

Year	Coastal	Unmonitore	Monitored	Coastal	Unmonitore	Monitored 	 	P

Catchment 	area	d	inland	area	d	inland

inland	inland

Belgium

Data from	2000

Meuse	0	5939	0	0	1014	0	5939	1014

Rhine	0	0	0	0	0	0	0	0

Scheldt	0	2140	0	0	271	0	2140	271

Seine	0	0	0	0	0	0	0	0

Sum	2000	0	8079	0	0	1285	0	8079	1285

Riverine load summary report - national figures (tonnes/year)

Country	Specific 	Riverine load,	Riverine load,	Riverine load,	Riverine load,	Transboundar	Transboundar

Year	annual 			 		y riverine 	y riverine

Catchment 	riverine flow	without 	without 	flow-	flow-	inputs, 	inputs,

normalisation	normalisation	normalised 	normalised

mill m3/year	 tonnes/yr 	 tonnes/yr 	tonnes/yr 	tonnes/yr 	tonnes/yr 	tonnes/yr

Belgium

Data from	2000

Meuse	5500	0	0	0	0	9384	448

Rhine	0	0	0	0	0	0	0

Scheldt	4245	4670	52286	0	0	48968	3722

Seine	0	0	0	0	0	0	0

Sum	2000	9745	4670	52286	0	0	58352	4170

All anthropogenic sources nitrogen (tonnes/year)

Country	WWTP	Industry	Aquaculture	Households	Diffuse 	Sum

Year	sources

Belgium

2000	30668,09	4324,06	84	1350,11	0	36426,26

All anthropogenic sources phosphorous (tonnes/year)

Country	WWTP	Industry	Aquaculture	Households	Diffuse 	Sum

Year	sources

Belgium

2000	7309,64	602,26	13,9	203,73	0	8129,53

Aquaculture summary report

Country	Tot- N	Tot-N 	Tot- N	Tot-P 	Tot-P	Tot-P	Tot-N 	Tot-P

Year	Coastal 	Inland 	 Inland 	Coasta	 Inland 	Inland

Catchment 		monitore	unmonitore	l 	monitore	unmonitore

area

Belgium

Data from	2000

Whole country	0	84	0	0	13,9	0	84	13,9

Sum	2000	0	84	0	0	13,9	0	84	13,9

Sewage works and sewerage summary report (tonnes/year)

Country	Tot-N 	Tot-N	Tot-N	Tot-P	Tot- P	Tot-P	Tot-N 	Tot-P

Year	Coastal	 Inland	Inland	Coastal	 Inland 	 Inland

Catchment name	area	monitored	unmonitored	area	monitored	unmonitore

d

Belgium

Data from	2000

Meuse	0	6734,01	0	0	1024,84	0	6734,01	1024,84

Rhine	0	71	0	0	10	0	71	10

Scheldt	0	23862,08	0	0	3274,8	0	23862,08	6274,8

Seine	0	1	0	0	0	0	1	0

Sum	2000	0	30668,09	0	0	4309,64	0	30668,09	7309,64

Households not connected to public sewerage summary report - national figures (tonnes/year)

Country	Tot-N 	Tot-N	Tot-N	Tot-P	Tot-P	Tot-P	Tot-N 	Tot-P

Year	Coastal 	 Inland	Inland	Coastal 	 Inland 	 Inland

Catchment name	area	monitored	unmonitored	area 	monitored	unmonitore

d

Data from	2000

Meuse	0	348,67	0	0	40,9	0	348,67	40,9

Rhine	0	22	0	0	2	0	22	2

Scheldt	0	978,44	0	0	160,83	0	978,44	160,83

Seine	0	1	0	0	0	0	1	0

Sum	2000	0	1350,11	0	0	203,73	0	1350,11	203,73

Industrial plants report – National figures (tonnes/year)

Country	Tot-N 	Tot-N	Tot-N	Tot-P	Tot-P	Tot-P	Tot-N 	Tot-P

Year	Coastal	 Inland	Inland	Coastal	 Inland 	 Inland

Catchment 	area	monitored	unmonitored	area	monitored	unmonitore

name	d

Data from	2000

Catchment area	Meuse

Cookeries/refineries	0	103,83	0	0	0,2	0	103,83	0,2

Fertiliser industry	0	4,92	0	0	108,05	0	4,92	108,05

Food/drink related industry	0	119,08	0	0	24,9	0	119,08	24,9

Org. chemical/bio-chemical 	0	56,16	0	0	5,63	0	56,16	5,63

Other sectors	0	200,22	0	0	6,84	0	200,22	6,84

Pulp/paper industry	0	100,75	0	0	11,61	0	100,75	11,61

Waste processing industry	0	75,59	0	0	1,16	0	75,59	1,16

Sum	Meuse	0	660,55	0	0	158,39	0	660,55	158,39

Catchment area	Scheldt

Cookeries/refineries	0	237,04	0	0	7,36	0	237,04	7,36

Fertiliser industry	0	341,46	0	0	12,73	0	341,46	12,73

Food/drink related industry	0	422,29	0	0	187,5	0	422,29	187,5

Org. chemical/bio-chemical 	0	1529,34	0	0	150,89	0	1529,34	150,89

Other sectors	0	1008,54	0	0	82,63	0	1008,54	82,63

Industry

Pulp/paper industry	0	1,4	0	0	-0,9	0	1,4	-0,9

Waste processing industry	0	123,44	0	0	3,66	0	123,44	3,66

Sum	Scheldt	0	3663,51	0	0	443,87	0	3663,51	443,87

Sum	2000	0	4324,06	0	0	602,26	0	4324,06	602,26

Diffuse losses summary report  (surface water) – national figures (tonnes/year))

Country	Normalised 	 Tot-N	Tot-P	Tot-N 	Tot-P	Tot- N	Tot-P	 Tot-N	 Tot-P	Tot-N	Tot-P

Year	or	Agric	Agric	Dir.  	 Dir. 	Other	 Other	Backgr	Backgr	 (Agric 	 (Agric +

Catchment 	dep.	dep.	+

Dir. dep 	 Dir. dep

+	+

Other -	 Other -

Belgium

Data from	2000

Meuse	14098,7	775,1	0	0	0	0	0	0	14098,7	775,1

Rhine	449	36	0	0	0	0	0	0	449	36

Scheldt	31012,2	1501,85	0	0	0	0	0	0	31012,2	1501,85

Seine	0	0	0	0	0	0	0	0	0	0

Sum	2000	45559,9	2312,95	0	0	0	0	0	0	45559,9	2312,95

Sent by e-mail:

1985-N-Agriculture= 39580 t

1985-N-Industry=29280 t

1985-N-Households=31960 t

1995-N-Agriculture=36386 t

1995-N-Industry=15274 t

1995-N-Households=31764 t

1985-P-Agriculture=2470 t

1985-P-Industry=5460 t

1985-P-Households=9870 t

1995-P-Agriculture=1680 t

1995-P-Industry=3370 t

1995-P-Households=4810 t

Note that the figures for 1995 have been verified in 1998 (because we

reported estimations made in 1993 to the Fourth North Sea Conference) but

the P figures for 1995 have not been verified after 1995.

ALL the Belgian data are year specific. Belgium does not carries out the normalisation procedure (because this procedure is only optional in the RID Programme of OSPAR if my understanding is right).

As far as the HARP data are concerned, because the HARP Guideline makes reference to the RID, we understand that providing normalized data is an option only (the non option is the agreement to provide year specific data).

# Report from Denmark

## Catchment description summary report – national figures

Catchment name	Catchment 	Lake 	River 	Total 	Long term

Water 	Water 	Populatio

Area	Surface 	Surface	n	annual

flow

Mar 11-12	1810	19,62	0	122610	846

Mar 13	3485	37,54	0	149334	1612

Mar 14-15	341	2,02	0	9649	68

Mar 16	5180	17,49	0	321396	2239

Mar 2	1098	2,47	0	75355	369

Mar 30	139	0	0	4475	41

Mar 31-32-33	2079	7,74	0	435672	401

Mar 34	725	7,73	0	42253	203

Mar 35	3498	74,67	0	344055	1343

Mar 36	743	2,98	0	36617	196

Mar 37	7608	92,13	0	542440	2556

Mar 38-39	1057	1,97	0	93320	376

Sum	Denmark	27763	266,36	0	2177176	10250

Retention in surface waters summary report – national figures (tonnes/year)

Country	Tot-N	Tot-N	Tot-N	Tot-P	Tot-P	Tot-P 	Tot-N	Tot-

Year	Coastal	Unmonitore	Monitored	Coastal	Unmonitore	Monitored 	 	P

Catchment 	area	d	inland	area	d	inland

inland	inland

Denmark

Data from	2000

Mar 11-12	0	0	283	0	0	1,9	0	0

Mar 13	0	0	542	0	0	3,6	0	0

Mar 14-15	0	0	29	0	0	0,2	0	0

Mar 16	0	0	264	0	0	1,6	0	0

Mar 2	0	0	36	0	0	0,2	0	0

Mar 30	0	0	0	0	0	0	0	0

Mar 31-32-33	0	0	263	0	0	-5,5	0	0

Mar 34	0	0	112	0	0	0,7	0	0

Mar 35	0	0	1201	0	0	9,1	0	0

Mar 36	0	0	43	0	0	0,3	0	0

Mar 37	0	0	1350	0	0	9	0	0

Mar 38-39	0	0	28	0	0	0,2	0	0

Sum	2000	0	0	4151	0	0	21,3	0	0

Riverine load summary report - national figures (tonnes/year)

Country	Specific 	Riverine load,	Riverine load,	Riverine load,	Riverine load,	Transboundar	Transboundar

Year	annual 			 		y riverine 	y riverine

Catchment 	riverine flow	without 	without 	flow-	flow-	inputs, 	inputs,

normalisation	normalisation	normalised 	normalised

mill m3/year	 tonnes/yr 	 tonnes/yr 	tonnes/yr 	tonnes/yr 	tonnes/yr 	tonnes/yr

Denmark

Data from	1991

Mar 11-12	811	77	3251	89	3766	0	0

Mar 13	1466	130	5529	151	6404	0	0

Mar 14-15	52	4	569	4,6	659	0	0

Mar 16	1916	303	9497	351	11000	0	0

Mar 2	292	94	1985	118	2498	0	0

Mar 30	33	2	186	2	210	0	0

Mar 31-32-33	447	150	3069	169	3463	0	0

Mar 34	138	15	859	17	969	0	0

Mar 35	1174	148	4529	167	5111	0	0

Mar 36	164	17	990	19	1117	0	0

Mar 37	2294	327	15633	369	17642	0	0

Mar 38-39	283	41	1796	46	2027	0	0

Sum	1991	9070	1308	47893	1502,6	54866	0	0

Data from	1992

Mar 11-12	827	69	4165	75	4515	0	0

13

Mar 13	1668	128	7436	139	8161	0	0

Mar 14-15	69	7	803	7,6	871	0	0

Mar 16	1990	252	11883	273	12882	0	0

Mar 2	297	80	2303	98	2834	0	0

Mar 30	32	2	221	23	252	0	0

Mar 31-32-33	375	96	3312	109	3777	0	0

Mar 34	120	10	853	11	973	0	0

Mar 35	1241	124	4982	141	5681	0	0

Mar 36	152	22	1532	25	1747	0	0

Mar 37	2286	332	17861	379	20369	0	0

Mar 38-39	287	30	2056	34	2345	0	0

Sum	1992	9344	1152	57407	1314,6	64407	0	0

Data from	1993

Mar 11-12	770	84	3256	90	3488	0	0

Mar 13	1519	117	6110	125	6546	0	0

Mar 14-15	60	9	542	10	581	0	0

Mar 16	2257	257	11583	275	12409	0	0

Mar 2	269	62	1998	84	2709	0	0

Mar 30	34	4	238	45	268	0	0

Mar 31-32-33	380	121	3972	136	4467	0	0

Mar 34	168	13	862	14	969	0	0

Mar 35	1254	121	5299	136	5959	0	0

Mar 36	163	28	1675	31	1884	0	0

Mar 37	2243	304	15789	342	17756	0	0

13.

Mar 38-39	311	40	2169	45	2439	0	0

Sum	1993	9428	1160	53493	1333	59475	0	0

Data from	1994

Mar 11-12	1027	130	4599	104	3672	0	0

Mar 13	2022	172	7763	137	6199	0	0

Mar 14-15	111	37	863	30	689	0	0

Mar 16	3019	344	13941	275	11132	0	0

Mar 2	466	121	3165	95	2479	0	0

Mar 30	53	10	342	76	261	0	0

Mar 31-32-33	633	130	5161	99	3942	0	0

Mar 34	252	25	1325	19	1012	0	0

Mar 35	1736	194	7408	148	5659	0	0

Mar 36	234	49	2082	37	1590	0	0

Mar 37	3302	508	21378	388	16331	0	0

Mar 38-39	501	92	3148	70	2405	0	0

Sum	1994	13356	1812	71175	1478	55371	0	0

Data from	1995

Mar 11-12	880	70	3303	64	3042	0	0

Mar 13	1845	150	6329	138	5830	0	0

Mar 14-15	90	29	419	27	386	0	0

Mar 16	2541	300	10935	276	10072	0	0

Mar 2	363	87	2300	87	2313	0	0

Mar 30	42	6	202	6	187	0	0

Mar 31-32-33	458	105	3455	97	3202	0	0

Mar 34	208	19	1137	18	1054	0	0

Mar 35	1534	154	5824	143	5398	0	0

Mar 36	235	34	1845	32	1710	0	0

Mar 37	2677	369	17320	342	16054	0	0

Mar 38-39	378	64	1920	57	1780	0	0

Sum	1995	11251	1387	54989	1287	51028	0	0

Data from	1996

Mar 11-12	504	31	1853	54	3222	0	0

Mar 13	1030	89	3900	155	6780	0	0

Mar 14-15	19	1,6	277	2,8	482	0	0

Mar 16	1284	157	5836	273	10147	0	0

Mar 2	237	54	1590	80	2369	0	0

Mar 30	32	4	142	6	211	0	0

Mar 31-32-33	131	47	674	70	1000	0	0

Mar 34	142	16	858	24	1273	0	0

Mar 35	965	92	3060	122	4542	0	0

Mar 36	166	24	1310	36	1944	0	0

Mar 37	1746	262	10507	389	15594	0	0

Mar 38-39	272	38	1336	56	1983	0	0

Sum	1996	6528	815,6	31343	1267,8	49547	0	0

Data from	1997

Mar 11-12	572	39	2044	62	3253	0	0

Mar 13	1093	74	3385	118	5387	0	0

Mar 14-15	18	2,7	297	4,3	473	0	0

Mar 16	1417	163	6152	259	9790	0	0

Mar 2	245	52	1481	77	2207	0	0

Mar 30	31	4	140	6	199	0	0

Mar 31-32-33	178	44	885	62	1257	0	0

Mar 34	141	15	949	21	1347	0	0

Mar 35	1040	113	3183	160	4520	0	0

Mar 36	164	21	1162	30	1650	0	0

Mar 37	1790	257	11387	365	16168	0	0

Mar 38-39	266	41	1303	58	1850	0	0

Sum	1997	6955	825,7	32368	1222,3	48101	0	0

Data from	1998

Mar 11-12	862	67	3368	66	3302	0	0

Mar 13	1574	113	5530	111	5422	0	0

Mar 14-15	61	10	363	9,8	356	0	0

Mar 16	2535	288	12039	282	11804	0	0

Mar 2	415	66	2863	58	2517	0	0

Mar 30	40	6	287	6	273	0	0

Mar 31-32-33	516	82	4044	78	3845	0	0

Mar 34	235	18	1291	17	1227	0	0

Mar 35	1362	154	5452	146	5183	0	0

Mar 36	189	31	1643	29	1562	0	0

Mar 37	263	347	17171	330	16324	0	0

13

Mar 38-39	418	64	2710	61	2576	0	0

Sum	1998	8470	1246	56761	1193,8	54391	0	0

Data from	1999

Mar 11-12	1127	98	4055	80	3303	0	0

Mar 13	1864	160	6086	130	4958	0	0

Mar 14-15	118	12	273	9,8	222	0	0

Mar 16	2937	413	14092	336	11479	0	0

Mar 2	564	128	3129	83	2025	0

Mar 30	60	12	295	9	230	0	0

Mar 31-32-33	481	76	3128	59	2443	0	0

Mar 34	350	24	1554	19	1214	0	0

Mar 35	1580	187	6908	146	5394	0	0

Mar 36	255	46	1968	36	1537	0	0

Mar 37	3298	479	19552	374	15268	0	0

Mar 38-39	541	113	2712	88	2118	0	0

Sum	1999	13175	1748	63752	1369,8	50191	0	0

Data from	2000

Mar 11-12	1084	112	3992	97	3463	0	0

Mar 13	2037	146	6544	127	5676	0	0

Mar 14-15	73	9	225	8	195	0	0

Mar 16	2494	278	10509	241	9116	0	0

Mar 2	539	113	2782	76	1882	0	0

Mar 30	57	9	238	6	161	0	0

Mar 31-32-33	414	71	2418	58	1963	0	0

Mar 34	271	9	896	7	727	0	0

Mar 35	1545	176	6027	143	4893	0	0

Mar 36	239	39	1774	32	1440	0	0

Mar 37	3290	483	19324	392	15689	0	0

Mar 38-39	498	87	2183	71	1772	0	0

Sum	2000	12541	1532	56912	1258	46977	0	0

# All anthropogenic sources nitrogen (tonnes/year)

Country	WWTP	Industry	Aquaculture	Households	Diffuse 	Sum

Year	sources

Denmark

2000	1980,8	483,984	1106,047	1253,8	38166,8	42991,431

# All anthropogenic sources phosphorous (tonnes/year)

Country	WWTP	Industry	Aquaculture	Households	Diffuse 	Sum

Year	sources

Denmark

2000	207,2	38,3694	84,659	287,85	987,09	1605,1684

d

# Aquaculture summary report

Country	Tot- N	Tot-N 	Tot- N	Tot-P 	Tot-P	Tot-P	Tot-N 	Tot-P

Year	Coastal 	Inland 	 Inland 	Coasta	 Inland 	Inland

Catchment 		monitore	unmonitore	l 	monitore	unmonitore

area

Denmark

Data from	2000

Mar 11-12	0	96,236	0	0	6,201	0	96,236	6,201

Mar 13	45,8	378,668	0	4,1	28,971	0	378,714	28,975

Mar 14-15	0	0	0	0	0	0	0	0

Mar 16	0	283,226	0	0	22,265	0	283,226	22,265

Mar 2	2,7	10,026	0,9	0,733	0	12,726	1,633

Mar 30	0	0	0	0	0	0	0	0

Mar 31-32-33	0	0	0	0	0	0	0	0

Mar 34	0	0	0	0	0	0	0	0

Mar 35	0	90,808	0	0	6,49	0	90,808	6,49

Mar 36	0	21,43	0	0	1,942	0	21,43	1,942

Mar 37	0,3	217,224	0	0	16,717	0	217,524	16,717

Mar 38-39	0	5,383	0	0	0,436	0	5,383	0,436

Sum	2000	48,8	1103,001	0	5	83,755	0	1106,047	84,659

Sewage works and sewerage summary report (tonnes/year)	 national /year)

Country	Tot-N 	Tot-N	Tot-N	Tot-P	Tot- P	Tot-P	Tot-N 	Tot-P

Year	Coastal	 Inland	Inland	Coastal	 Inland 	 Inland

Catchment name	area	monitored	unmonitored	area	monitored	unmonitore

d

Denmark

Data from	2000

Mar 11-12	0,7	155,5	0	0,1	12,3	0	156,2	12,4

Mar 13	9,4	116,2	0	1,2	9,7	0	125,6	10,9

Mar 14-15	0	0,8	0	0	0,1	0	0,8	0,1

Mar 16	94,1	289,2	0	3,8	41,8	0	383,3	45,6

Mar 2	18,4	175,2	0	5,1	11,4	0	193,6	16,5

Mar 30	5,8	0	0	1,3	0	0	5,8	1,3

Mar 31-32-33	74,1	151,3	0	14	24,4	0	225,4	38,4

Mar 34	19,3	27,4	0	2,1	4,2	0	46,7	6,3

Mar 35	3,1	328,3	0	0,2	23,1	0	331,4	23,3

Mar 36	45,4	12,2	0	1,6	1,3	0	57,6	2,9

Mar 37	202,1	252,3	0	22,7	26,8	0	454,4	49,5

Mar 38-39	93,1	15	0	12,7	2,7	0	0	0

Sum	2000	565,5	1523,4	0	64,8	157,8	0	1980,8	207,2

Households not connected to public sewerage summary report - national figures (tonnes/year)

Country	Tot-N 	Tot-N	Tot-N	Tot-P	Tot-P	Tot-P	Tot-N 	Tot-P

Year	Coastal 	 Inland	Inland	Coastal 	 Inland 	 Inland

Catchment name	area	monitored	unmonitored	area 	monitored	unmonitore

d

Denmark

Data from	2000

Mar 11-12	0	28,6	0	0	6,5	0	28,6	6,5

Mar 13	0	52,9	0	0	12	0	52,9	12

Mar 14-15	0	2,7	0	0	0,6	0	2,7	0,6

Mar 16	0	85,1	0	0	19,7	0	85,1	19,7

Mar 2	0	16,5	0	0	3,7	0	16,5	3,7

Mar 30	0	0,2	0	0	0,05	0	0,2	0,05

Mar 31-32-33	810	40,1	0	185	9,2	0	850,1	194,2

Mar 34	0	10,8	0	0	2,4	0	10,8	2,4

Mar 35	0	58,8	0	0	13,4	0	58,8	13,4

Mar 36	0	11,8	0	0	2,7	0	11,8	2,7

Mar 37	0	127	0	0	28,8	0	127	28,8

Mar 38-39	0	9,3	0	0	3,8	0	9,3	3,8

Sum	2000	810	443,8	0	185	102,85	0	1253,8	287,85

Industrial plants report – National figures (tonnes/year)ar)

Country	Tot-N 	Tot-N	Tot-N	Tot-P	Tot-P	Tot-P	Tot-N 	Tot-P

Year	Coastal	 Inland	Inland	Coastal	 Inland 	 Inland

Catchment 	area	monitored	unmonitored	area	monitored	unmonitore

name	d

Industry

Denmark

Data from	2000

Catchment area	Mar 11-12

Org. chemical/bio-chemical 	31,3	0	0	0	9,9	0	31,3	9,9

Sum	Mar 11-12	31,3	0	0	0	9,9	0	31,3	9,9

Catchment area	Mar 13

Food/drink related industry	0	3,7	0	0	0,76	0	3,7	0,76

Other sectors	0	4,2	0	0	0,84	0	4,2	0,84

Pulp/paper industry	0	1,18	0	0	0,08	0	1,18	0,08

Sum	Mar 13	0	9,08	0	0	1,68	0	9,08	1,68

Catchment area	Mar 14-15

Other sectors	0	0,25	0	0	0,006	0	0,25	0,006

Sum	Mar 14-15	0	0,25	0	0	0,006	0	0,25	0,006

Catchment area	Mar 16

Food/drink related industry	91,8	8,5	0	0,52	1,13	0	100,3	1,65

12. mars 2002	Page 1 of 3

Country	Tot-N 	Tot-N	Tot-N	Tot-P	Tot-P	Tot-P	Tot-N 	Tot-P

Year	Coastal	 Inland	Inland	Coastal	 Inland 	 Inland

Catchment 	area	monitored	unmonitored	area	monitored	unmonitore

name	d

Industry

Waste processing industry	0	0	0	0	0,009	0	0	0,009

Sum	Mar 16	91,8	8,5	0	0,52	1,139	0	100,3	1,659

Catchment area	Mar 2

Food/drink related industry	55,2	0	0	5,6	0	0	55,2	5,6

Other sectors	0	0,125	0	0	0	0	0,125	0

Sum	Mar 2	55,2	0,125	0	5,6	0	0	55,325	5,6

Catchment area	Mar 30

0	0	0	0	0	0	0	0

Sum	Mar 30	0	0	0	0	0	0	0	0

Catchment area	Mar 31-32-33

Food/drink related industry	0,2	0	0	0,9	0	0	0,2	0,09

Org. chemical/bio-chemical 	0,18	0,7	0	0,001	0	0	0,88	0,001

Other sectors	0	3,72	0	0	0,136	0	3,72	0,136

Sum	Mar 31-32-33	0,38	4,42	0	0,901	0,136	0	4,8	0,227

Catchment area	Mar 34

Food/drink related industry	4,47	0	0	0,19	0	0	4,47	0,19

Org. chemical/bio-chemical 	14,9	0	0	2,6	0	0	14,9	2,6

Sum	Mar 34	19,37	0	0	2,79	0	0	19,37	2,79

Catchment area	Mar 35

Food/drink related industry	16,1	0	0	0,155	0	0	16,1	0,155

12. mars 2002	Page 2 of 3

Country	Tot-N 	Tot-N	Tot-N	Tot-P	Tot-P	Tot-P	Tot-N 	Tot-P

Year	Coastal	 Inland	Inland	Coastal	 Inland 	 Inland

Catchment 	area	monitored	unmonitored	area	monitored	unmonitore

name	d

Industry

Pulp/paper industry	0,513	0	0	0,0054	0	0	0,513	0,0054

Sum	Mar 35	16,613	0	0	0,1604	0	0	16,613	0,1604

Catchment area	Mar 36

Other sectors	0,256	0	0	0	0	0	0,256	0

Sum	Mar 36	0,256	0	0	0	0	0	0,256	0

Catchment area	Mar 37

Food/drink related industry	59,3	0	0	5,6	0	0	59,3	5,6

Other sectors	4,23	0	0	1,157	0	0	4,23	1,157

Sum	Mar 37	63,53	0	0	6,757	0	0	63,53	6,757

Catchment area	Mar 38-39

Food/drink related industry	183,16	0	0	9,59	0	0	183,16	9,59

Sum	Mar 38-39	183,16	0	0	9,59	0	0	183,16	9,59

Sum	2000	461,609	22,375	0	26,3184	12,861	0	483,984	38,3694

Diffuse losses summary report  (surface water) – national figures (tonnes/year))

Country	Normalised 	 Tot-N	Tot-P	Tot-N 	Tot-P	Tot- N	Tot-P	 Tot-N	 Tot-P	Tot-N	Tot-P

Year	or	Agric	Agric	Dir.  	 Dir. 	Other	 Other	Backgr	Backgr	 (Agric 	 (Agric +

Catchment 	dep.	dep.	+

Dir. dep 	 Dir. dep

+	+

Other -	 Other -

Denmark

Data from	2000

Mar 11-12	Year specific	3973	82	0	0	0	0	391	13,6	3582	68,4

Mar 11-12	Normalised	3470	77	0	0	0	0	339	11,8	3131	65,2

Mar 13	Normalised	5701	91	0	0	0	0	653	22,5	5048	68,5

Mar 13	Year specific	6519	93	0	0	0	0	753	26	5766	67

Mar 14-15	Normalised	219	8,7	0	0	0	0	66,2	2,3	152,8	6,4

Mar 14-15	Year specific	250	8,9	0	0	0	0	76,3	2,6	173,7	6,3

Mar 16	Normalised	8782	170	0	0	0	0	978	34	7804	136

Mar 16	Year specific	10038	176	0	0	0	0	1127	39	8911	137

Mar 2	Normalised	1772	66	0	0	0	0	160	5,5	1612	60,5

Mar 2	Year specific	2602	98	0	0	0	0	237	8,2	2365	89,8

Mar 30	Normalised	193	7,3	0	0	0	0	24	0,81	169	6,49

Mar 30	Year specific	238	9	0	0	0	0	30	1	208	8

Mar 31-32-33	Normalised	2681	176	0	0	0	0	365	12,7	2316	163,3

Mar 31-32-33	Year specific	2450	23	0	0	0	0	450	15,6	2000	7,4

Mar 34	Normalised	792	3,2	0	0	0	0	127	4,5	665	-1,3

Mar 34	Year specific	868	1,9	0	0	0	0	157	5,5	711	-3,6

12. mars 2002	Page 1 of 2

Country	Normalised 	 Tot-N	Tot-P	Tot-N 	Tot-P	Tot- N	Tot-P	 Tot-N	 Tot-P	Tot-N	Tot-P

Year	or	Agric	Agric	Dir.  	 Dir. 	Other	 Other	Backgr	Backgr	 (Agric 	 (Agric +

Catchment 	dep.	dep.	+

Dir. dep 	 Dir. dep

+	+

Other -	 Other -

Mar 35	Normalised	5488	116	0	0	0	0	6140	21	-652	95

Mar 35	Year specific	6701	130	0	0	0	0	756	26	5945	104

Mar 36	Normalised	1443	28	0	0	0	0	131	4,5	1312	23,5

Mar 36	Year specific	1765	32	0	0	0	0	161	5,6	1604	26,4

Mar 37	Year specific	20000	400	0	0	0	0	1624	56	18376	344

Mar 37	Normalised	16341	348	0	0	0	0	1319	45	15022	303

Mar 38-39	Normalised	1773	67	0	0	0	0	186	6,5	1587	60,5

Mar 38-39	Year specific	2168	79	0	0	0	0	229	8	1939	71

Sum	2000	106227	2291	0	0	0	0	16479,5	378,21	89747,5	1912,79

| Spildevand: Danmark |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | KVÆLSTOF |  |  |  |  |  |  | FOSFOR |  |  |  |
| Ferskvand |  |  | Regnvands- | Spredt |  |  | Ferskvand |  |  | Regnvands- | Spredt |  |  |
| ton | Renseanlæg | Industri | betinget | bebyggelse | Dambrug | I alt | ton | Renseanlæg | Industri | betinget | bebyggelse | Dambrug | I alt |
| 1981-88 | 6700 | 240 | 570 | 1610 | 2200 | 11320 | 1981-88 | 2100 | 100 | 170 | 470 | 400 | 3240 |
| 1989 | 6700 | 236 | 570 | 1595 | 2189 | 11290 | 1989 | 1770 | 18 | 140 | 460 | 239 | 2627 |
| 1990 | 6341 | 225 | 850 | 1250 | 1952 | 10618 | 1990 | 1323 | 18 | 210 | 420 | 216 | 2187 |
| 1991 | 5961 | 152 | 685 | 1061 | 1729 | 9588 | 1991 | 961 | 14 | 182 | 371 | 169 | 1697 |
| 1992 | 5311 | 106 | 620 | 1245 | 1535 | 8817 | 1992 | 742 | 6 | 156 | 286 | 140 | 1330 |
| 1993 | 4119 | 108 | 711 | 1296 | 1372 | 7606 | 1993 | 524 | 7 | 179 | 297 | 115 | 1122 |
| 1994 | 3961 | 96 | 836 | 1198 | 1378 | 7469 | 1994 | 454 | 7 | 212 | 273 | 119 | 1065 |
| 1995 | 3451 | 121 | 619 | 1137 | 1385 | 6713 | 1995 | 410 | 4 | 158 | 261 | 106 | 939 |
| 1996 | 2763 | 104 | 443 | 1140 | 1212 | 5662 | 1996 | 324 | 3 | 112 | 262 | 93 | 794 |
| 1997 | 2436 | 55 | 569 | 1121 | 1227 | 5408 | 1997 | 263 | 3 | 145 | 258 | 92 | 761 |
| 1998 | 2710 | 66 | 723 | 989 | 1241 | 11317 | 1998 | 274 | 4 | 190 | 226 | 92 | 1546 |
| 1999 | 2705 | 38 | 752 | 966 | 1127 | 5588 | 1999 | 258 | 4 | 195 | 220 | 83 | 760 |
| 2000 | 2475 | 27 | 592 | 976 | 1195 | 5265 | 2000 | 248 | 4 | 150 | 222 | 91 | 715 |

| Direkte |  |  | Regnvands- | Spredt |  |  | Direkte |  |  | Regnvands- | Spredt |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ton | Renseanlæg | Industri | betinget | bebyggelse | Havbrug | I alt | ton | Renseanlæg | Industri | betinget | bebyggelse | Havbrug | I alt |
| 1981-88 | 12300 | 4760 | 430 | 20 | 300 | 17810 | 1981-88 | 3900 | 3300 | 90 | 5 | 40 | 7335 |
| 1989 | 11300 | 4742 | 240 | 12 | 322 | 16616 | 1989 | 2700 | 1177 | 59 | 5 | 44 | 3985 |
| 1990 | 10544 | 3862 | 232 | 12 | 332 | 14982 | 1990 | 2391 | 632 | 56 | 3 | 40 | 3122 |
| 1991 | 9148 | 3845 | 235 | 12 | 266 | 13506 | 1991 | 1838 | 563 | 62 | 3 | 36 | 2502 |
| 1992 | 7760 | 4079 | 261 | 12 | 361 | 22297 | 1992 | 1521 | 418 | 67 | 3 | 37 | 3642 |
| 1993 | 6666 | 2467 | 314 | 12 | 365 | 9824 | 1993 | 1238 | 238 | 78 | 3 | 39 | 1596 |
| 1994 | 6280 | 2641 | 371 | 12 | 306 | 9610 | 1994 | 1120 | 313 | 94 | 3 | 32 | 1562 |
| 1995 | 5487 | 2350 | 248 | 4 | 350 | 8439 | 1995 | 827 | 201 | 65 | 1 | 37 | 1131 |
| 1996 | 3642 | 1625 | 185 | 3 | 331 | 5786 | 1996 | 580 | 117 | 47 | 0 | 35 | 779 |
| 1997 | 2420 | 1745 | 232 | 1 | 269 | 4667 | 1997 | 403 | 142 | 57 | 0 | 30 | 632 |
| 1998 | 2454 | 1363 | 242 | 3 | 291 | 4353 | 1998 | 327 | 121 | 62 | 1 | 33 | 544 |
| 1999 | 2429 | 824 | 223 | 4 | 291 | 3771 | 1999 | 324 | 66 | 56 | 1 | 33 | 480 |
| 2000 | 2179 | 871 | 169 | 5 | 326 | 3550 | 2000 | 295 | 54 | 43 | 1 | 36 | 429 |

| Spildevand i alt |  |  | Regnvands- | Spredt |  |  | Spildevand i alt |  |  | Regnvands- | Spredt |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ton | Renseanlæg | Industri | betinget | bebyggelse | Dambrug | I alt | ton | Renseanlæg | Industri | betinget | bebyggelse | Dambrug | I alt |
| 1981-88 | 19000 | 5000 | 1000 | 1630 | 2500 | 29130 | 1981-88 | 6000 | 3400 | 260 | 475 | 440 | 10575 |
| 1989 | 18000 | 4978 | 810 | 1607 | 2511 | 27906 | 1989 | 4470 | 1195 | 199 | 465 | 283 | 6612 |
| 1990 | 16885 | 4087 | 1082 | 1262 | 2284 | 25600 | 1990 | 3714 | 650 | 266 | 423 | 256 | 5309 |
| 1991 | 15109 | 3997 | 920 | 1073 | 1995 | 23094 | 1991 | 2799 | 577 | 244 | 374 | 205 | 4199 |
| 1992 | 13071 | 4185 | 881 | 1257 | 1896 | 21290 | 1992 | 2263 | 424 | 223 | 289 | 177 | 3376 |
| 1993 | 10785 | 2575 | 1025 | 1308 | 1737 | 34509 | 1993 | 1762 | 245 | 257 | 300 | 154 | 5345 |
| 1994 | 10241 | 2737 | 1207 | 1210 | 1684 | 17079 | 1994 | 1574 | 320 | 306 | 276 | 151 | 2627 |
| 1995 | 8938 | 2471 | 867 | 1141 | 1735 | 15152 | 1995 | 1237 | 205 | 223 | 262 | 143 | 2070 |
| 1996 | 6405 | 1729 | 628 | 1143 | 1543 | 11448 | 1996 | 904 | 120 | 159 | 262 | 128 | 1573 |
| 1997 | 4856 | 1800 | 801 | 1122 | 1496 | 10075 | 1997 | 666 | 145 | 202 | 258 | 122 | 1393 |
| 1998 | 5164 | 1429 | 965 | 992 | 1532 | 10082 | 1998 | 601 | 125 | 252 | 227 | 125 | 1330 |
| 1999 | 5134 | 862 | 975 | 970 | 1418 | 9359 | 1999 | 582 | 70 | 251 | 221 | 116 | 1240 |
| 2000 | 4654 | 898 | 761 | 981 | 1521 | 8815 | 2000 | 543 | 58 | 193 | 223 | 127 | 1144 |

| Samlede udledninger via vandløb og direkte udledninger fra Danmark |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Fosfor |  |  |  | Q-vægtet | Q-vægtet |  | Havbrug | Total |
| hav (ton) | via vandløb | direkte | i alt | vandløb mgP/l | i alt mgP/l | kg/ha |  | inkl. havb. |
| 1981-88 | 4200 | 5750 | 9950 | 0,258 | 0,612 | 0,98 | 40 | 9990 |
| 1989 | 2862 | 3972 | 6834 | 0,265 | 0,633 | 0,67 | 44 | 6878 |
| 1990 | 3569 | 3099 | 6668 | 0,255 | 0,476 | 0,83 | 40 | 6708 |
| 1991 | 2327 | 2499 | 4826 | 0,183 | 0,380 | 0,54 | 36 | 4862 |
| 1992 | 1964 | 2048 | 4012 | 0,156 | 0,318 | 0,46 | 39 | 4051 |
| 1993 | 2039 | 1584 | 3623 | 0,146 | 0,259 | 0,47 | 38 | 3661 |
| 1994 | 2963 | 1529 | 4492 | 0,151 | 0,229 | 0,69 | 32 | 4524 |
| 1995 | 2190 | 1132 | 3322 | 0,140 | 0,213 | 0,51 | 37 | 3359 |
| 1996 | 1228 | 746 | 1974 | 0,150 | 0,241 | 0,29 | 35 | 2009 |
| 1997 | 1220 | 602 | 1822 | 0,137 | 0,205 | 0,28 | 30 | 1852 |
| 1998 | 2087 | 511 | 2598 | 0,134 | 0,167 | 0,49 | 33 | 2631 |
| 1999 | 2587 | 446 | 3033 | 0,141 | 0,165 | 0,60 | 33 | 3066 |
| 2000 | 2123 | 394 | 2517 | 0,129 | 0,153 | 0,49 | 35 | 2552 |
| 1989-2000 | 2263 | 1547 | 3810 | 0,163 | 0,274 | 0,53 | 36 | 3846 |

| Kvælstof |  |  |  | Q-vægtet | Q-vægtet |  | Havbrug | Total |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| hav (ton) | via vandløb | direkte | i alt | vandløb mgN/l | i alt mgN/l | kg/ha |  | inkl. havb. |
| 1981-88 | 109000 | 16700 | 125700 | 6,7 | 7,7 | 25,3 | 350 | 126050 |
| 1989 | 61883 | 16659 | 78542 | 5,7 | 7,3 | 14,4 | 322 | 78864 |
| 1990 | 97058 | 14890 | 111948 | 6,9 | 8,0 | 22,6 | 332 | 112280 |
| 1991 | 78468 | 13494 | 91962 | 6,2 | 7,2 | 18,2 | 265 | 92227 |
| 1992 | 91721 | 12459 | 104180 | 7,3 | 8,3 | 21,3 | 361 | 104541 |
| 1993 | 98163 | 9687 | 107840 | 7,0 | 7,7 | 22,8 | 366 | 108206 |
| 1994 | 119071 | 9299 | 128370 | 6,1 | 6,6 | 27,7 | 306 | 128676 |
| 1995 | 84400 | 8446 | 92846 | 5,4 | 5,9 | 19,6 | 351 | 93197 |
| 1996 | 42541 | 5445 | 47986 | 5,2 | 5,9 | 9,9 | 332 | 48318 |
| 1997 | 45427 | 4398 | 49825 | 5,1 | 5,6 | 10,6 | 267 | 50092 |
| 1998 | 96573 | 4062 | 100635 | 6,2 | 6,5 | 22,4 | 291 | 100926 |
| 1999 | 97711 | 3482 | 101193 | 5,3 | 5,5 | 22,7 | 291 | 101484 |
| 2000 | 79791 | 3245 | 83036 | 4,9 | 5,1 | 18,5 | 326 | 83362 |
| 1989-2000 | 82734 | 8797 | 91530 | 6,0 | 6,6 | 19,2 | 317,5 | 91848 |

# Report from Germany

Click on icon to open German reporting Excel file

# Report from Norway

Catchment description summary report - national figures

Catchment name	Catchment 	Lake 	River 	Total 	Long term

Water 	Water 	Populatio

Area	Surface 	Surface	n	annual

flow

Alta	56648	339	0	6450	7,487

Drammenselva	17539	652	0	22970	26,743

Glomma	47541	1943	0	133130	60,324

Numedalslågen	7253	254	0	19890	10,082

Orkla	45792	1703	0	63850	5,374

Orreelva	9765	525	0	25590	333

Otra	13755	607	0	19800	12,841

Skienselva	12902	1022	0	15700	22,611

Suldalslågen	28446	1034	0	42980	7,422

Vefsna	47387	1056	0	41020	15,62

Sum	Norway	287028	9135	0	391380	501,504

Retention in surface waters summary report - national figures (tonnes/year)

Tot-N	Tot-N	Tot-N	Tot-P	Tot-P	Tot-P 	Tot-N	Tot-

Year	Coastal	Unmonitore	Monitored	Coastal	Unmonitore	Monitored 	 P

Catchment 	area	d	inland	area	d	inland

inland	inland

| 1985 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Alta | 33,4 | 0,0 | 0,0 | 9,6 | 0,0 | 0,0 | 33,4 | 9,6 |
|  | Drammenselva | 0,0 | 0,0 | 832,5 | 0,9 | 0,0 | 146,0 | 832,5 | 146,9 |
|  | Glomma | 789,4 | 0,0 | 1753,0 | 113,4 | 0,0 | 350,1 | 2542,4 | 463,5 |
|  | Numedalslågen | 0,6 | 0,0 | 107,5 | 0,5 | 0,0 | 30,1 | 108,1 | 30,6 |
|  | Orkla | 418,3 | 0,0 | 0,0 | 88,3 | 0,0 | 0,0 | 418,3 | 88,3 |
|  | Orreelva | 240,3 | 0,0 | 28,9 | 33,4 | 0,0 | 9,4 | 269,2 | 42,8 |
|  | Otra | 238,6 | 0,0 | 144,4 | 36,5 | 0,0 | 12,6 | 383,0 | 49,1 |
|  | Skienselva | 34,2 | 0,0 | 737,3 | 0,2 | 0,0 | 105,0 | 771,5 | 105,2 |
|  | Suldalslågen | 211,5 | 0,0 | 0,0 | 26,8 | 0,0 | 0,0 | 211,5 | 26,8 |
|  | Vefsna | 73,7 | 0,0 | 5,4 | 32,7 | 0,0 | 1,2 | 79,1 | 33,9 |
|  |  | 2040,0 | 0,0 | 3609,1 | 342,3 | 0,0 | 654,5 | 5649,0 | 996,7 |
| 2000 |  |  |  |  |  |  |  |  |  |
|  | Alta | 35,0 | 0,0 | 0,0 | 9,3 | 0,0 | 0,0 | 35,0 | 9,3 |
|  | Drammenselva | 233,5 | 403,9 | 0,0 | 0,0 | 0,0 | 93,5 | 637,4 | 94,2 |
|  | Glomma | 644,5 | 1450,6 | 0,0 | 63,2 | 0,0 | 229,0 | 2095,1 | 291,7 |
|  | Numedalslågen | 5,4 | 94,0 | 0,0 | 4,5 | 0,0 | 18,8 | 99,4 | 21,9 |
|  | Orkla | 4301,9 | 0,0 | 0,0 | 69,6 | 0,0 | 0,0 | 4301,9 | 82,6 |
|  | Orreelva | 298,7 | 43,8 | 0,0 | 43,1 | 0,0 | 7,2 | 342,5 | 49,2 |
|  | Otra | 222,0 | 163,1 | 0,0 | 28,4 | 0,0 | 11,1 | 385,2 | 39,8 |
|  | Skienselva | 26,8 | 561,0 | 0,0 | 5,5 | 0,0 | 73,6 | 587,8 | 79,2 |
|  | Suldalslågen | 201,1 | 0,0 | 0,0 | 18,1 | 0,0 | 0,0 | 201,1 | 17,1 |
|  | Vefsna | 27,0 | 0,0 | 0,0 | 33,2 | 0,0 | 0,3 | 27,0 | 50,2 |
|  |  | 5995,9 | 2716,5 | 0,0 | 274,9 | 0,0 | 433,5 | 8712,4 | 735,1 |

Riverine load summary report - national figures (tonnes/year)

Country	Specific 	Riverine load,	Riverine load,	Riverine load,	Riverine load,	Transboundar	Transboundar

Year	annual 			 		y riverine 	y riverine

Catchment 	riverine flow	without 	without 	flow-	flow-	inputs, 	inputs,

normalisation	normalisation	normalised 	normalised

mill m3/year	 tonnes/yr 	 tonnes/yr 	tonnes/yr 	tonnes/yr 	tonnes/yr 	tonnes/yr

Norway

Data from	1985

Alta3276	31	622	25	519	0	0

Drammenselva	10686	108	3836	99	3504	0	0

Glomma	27427	485	14837	389	11911	0	0

Numedalslågen	3616	57	766	58	780	0	0

Orkla	2113	30	754	28	700	0	0

Orreelva	68	7	96	12	174	0	0

Otra	3988	35	1254	41	1474	0	0

Skienselva	7767	50	3106	53	3301	0	0

Suldalslågen	1705	5	376	8	598	0	0

Vefsna	4734	23	814	28	980	0	0

Sum	1985	65380	835	26466	746	23944

Data from	2000

Alta	3051	104	761	76	558	0	0

Drammenselva	13712	186	6212	137	4576	0	0

Glomma	31252	834	1773	577	12261	0	0

Numedalslågen	5267	255	2804	180	1980	0	0

Orkla	1672	34	647	32	627	0	0

Orreelva	151	14	320	14	330	0	0

Otra	6768	109	2017	79	1460	0	0

Skienselva	12718	84	4027	60	2881	0	0

Suldalslågen	1662	6	398	13	782	0	0

Vefsna	6014	23	657	23	655	0	0

Sum	2000	82267	1654	35578	1195	26114	0	0 2 of 2

All anthropogenic sources - tonnes N

Country	WWTP	Industry	Aquaculture	Households	Diffuse 	Sum

Year	sources

Norway

1985	16912	8038	1688	4533	37331	68502

2000	13164	3824	22901	3247	33625	76761

All anthropogenic sources - tonnes P

Country	WWTP	Industry	Aquaculture	Households	Diffuse 	Sum

Year	sources

Norway

1985	2082	361	355	718	1198	4714

2000	826	196	4823	342	1120	7307

Aquaculture summary report - national figures (tonnes/year)

Country	Tot- N	Tot-N 	Tot- N	Tot-P 	Tot-P	Tot-P	Tot-N 	Tot-P

Year	Coastal 	Inland 	 Inland 	Coasta	 Inland 	Inland

Catchment 		monitore	unmonitore	l 	monitore	unmonitore

area

Norway

Data from	1985

Alta	23,472	0	0	4,957	0	0	23,472	4,957

Drammenselva	0,255	0	0	0,051	0	0	0,255	0,051

Glomma	0,058	0	0	0,011	0	0	0,058	0,011

Numedalslågen	0	0	0	0	0	0	0	0

Orkla	524,117	0	0	110,081	0	0	524,117	110,081

Orreelva	56,964	0	0	12,058	0	0	56,964	12,058

Otra	1,205	0	0	0,26	0	0	1,205	0,26

Skienselva	0,098	0	0	0,019	0	0	0,098	0,019

Suldalslågen	647,933	0	0	136,555	0	0	647,933	136,555

Vefsna	433,985	0	0	91,574	0	0	433,985	91,574

Sum	1985	1688,087	0	0	355,566	0	0	1688,087	355,566

Data from	2000

Alta	1707,258	0	0	362,253	0	0	1707,258	362,253

Drammenselva	0	7,443	0	0	1,508	0	7,443	1,508

Glomma	0,098	2,235	0	0,019	0,425	0	2,333	0,444

Orkla	5582,58	0	0	1178,563	0	0	5582,58	1178,563

Orreelva	1263,224	0	0	267,717	0	0	1263,224	267,717

Otra	37,834	0	0	7,896	0	0	37,834	7,896

Skienselva	1,467	0,045	0,031	0,28	0,008	0,005	1,543	0,293

Suldalslågen	8365,965	0	0	1736,966	0	0	8365,965	1736,966

Vefsna	5933,377	0,166	0	1267,611	0,032	0	5933,543	1267,643

Sum	2000	22891,803	9,889	0,031	4821,305	1,973	0,005	22901,723	4823,283

Sewage works and sewerage summary report - national figures (tonnes/year)

Country	Tot-N 	Tot-N	Tot-N	Tot-P	Tot- P	Tot-P	Tot-N 	Tot-P

Year	Coastal	 Inland	Inland	Coastal	 Inland 	 Inland

Catchment name	area	monitored	unmonitored	area	monitored	unmonitore

d

Norway

Data from	1985

Alta	227	15	4	40	3	1	245	44

Drammenselva	149	455	332	25	37	37	936	100

Glomma	4063	2229	379	180	162	69	6672	412

Numedalslågen	994	187	0	173	22	0	1181	195

Orkla	1999	31	26	352	4	5	2056	361

Orreelva	914	74	0	167	14	0	989	180

Otra	742	124	0	123	18	0	866	141

Skienselva	233	326	297	33	30	55	857	118

Suldalslågen	1917	12	0	317	2	0	1928	319

Vefsna	1120	8	54	202	1	8	1182	212

Sum	1985	12358	3461	1092	1612	293	175	16912	2082

Data from	2000

Alta	368	10	0	46	1	0	378	47

Drammenselva	182	343	181	3	3	2	706	8

Glomma	2210	1629	370	46	16	7	4209	69

Numedalslågen	580	93	0	15	1	0	674	16

Orkla	2159	10	0	209	0	0	2169	210

Orreelva	681	88	52	52	1	7	821	60

Otra	595	26	0	32	0	0	620	32

Skienselva	122	126	231	2	4	3	479	9

Suldalslågen	1890	39	0	214	5	0	1930	219

Vefsna	1174	4	0	156	0	0	1178	156

Sum	2000	9961	2368	834	775	31	19	13164	826

Households not connected to public sewerage summary report - national figures

Country	Tot-N 	Tot-N	Tot-N	Tot-P	Tot-P	Tot-P	Tot-N 	Tot-P

Year	Coastal 	 Inland	Inland	Coastal 	 Inland 	 Inland

Catchment name	area	monitored	unmonitored	area 	monitored	unmonitore

d

Norway

Data from	1985

Alta	82	6	3	10	1	0	92	12

Drammenselva	27	260	19	4	35	3	305	42

Glomma	190	678	32	34	101	6	899	141

Numedalslågen	123	69	0	18	11	0	191	29

Orkla	955	20	10	158	3	1	984	163

Orreelva	238	14	0	40	2	0	253	43

Otra	205	31	0	32	5	0	236	37

Skienselva	38	91	19	6	14	3	148	22

Suldalslågen	752	8	0	124	1	0	759	125

Vefsna	653	8	5	102	1	1	666	104

Sum	1985	3263	1185	88	528	174	14	4533	718

Data from	2000

Alta	37	3	17	3	0	2	57	5

Drammenselva	15	173	10	2	12	1	198	14

Glomma	149	474	35	16	41	4	658	61

Numedalslågen	143	35	0	18	3	0	178	20

Orkla	550	12	10	64	1	1	572	66

Orreelva	148	11	0	15	1	0	160	16

Otra	149	19	0	14	1	0	167	16

Skienselva	26	68	4	3	7	0	98	10

Suldalslågen	618	8	0	68	1	0	626	69

Vefsna	513	7	13	62	1	2	533	65

Sum	2000	2348	810	89	265	68	10	3247	342

Industrial plants report - national figures (tonnes/year)

Country	Tot-N 	Tot-N	Tot-N	Tot-P	Tot-P	Tot-P	Tot-N 	Tot-P

Year	Coastal	 Inland	Inland	Coastal	 Inland 	 Inland

Catchment 	area	monitored	unmonitored	area	monitored	unmonitore

name	d

Industry

Norway

Data from	1985

Catchment area	Alta

Other sectors	1	0	0	1	0	0	1	1

Sum	Alta	1	0	0	1	0	0	1	1

Catchment area	Drammenselva

Other sectors	0	40	21	0	4	0	62	4

Sum	Drammenselva	0	40	21	0	4	0	62	4

Catchment area	Glomma

Other sectors	687	8	122	74	18	15	817	106

Sum	Glomma	687	8	122	74	18	15	817	106

Catchment area	Numedalslågen

Other sectors	42	0	0	2	0	0	42	2

Sum	Numedalslågen	42	0	0	2	0	0	42	2

Catchment area	Orkla

Other sectors	140	0	4	27	0	1	144	28

Sum	Orkla	140	0	4	27	0	1	144	28

Catchment area	Orreelva

Other sectors	48	9	0	2	1	0	57	3

Sum	Orreelva	48	9	0	2	1	0	57	3

Catchment area	Otra

Other sectors	0	39	0	2	8	0	39	10

Sum	Otra	0	39	0	2	8	0	39	10

Catchment area	Skienselva

Other sectors	4300	500	37	33	0	8	4837	41

Sum	Skienselva	4300	500	37	33	0	8	4837	41

Catchment area	Suldalslågen

Other sectors	658	0	0	29	0	0	659	29

Sum	Suldalslågen	658	0	0	29	0	0	659	29

Catchment area	Vefsna

Other sectors	1380	0	0	137	0	0	1380	137

Sum	Vefsna	1380	0	0	137	0	0	1380	137

Sum	1985	7256	596	184	307	31	24	8038	361

13

Data from	2000

Catchment area	Drammenselva

Other sectors	37	0	0	0	0	0	37	0

Pulp/paper industry	0	202	0	0	4	0	202	4

Sum	Drammenselva	37	202	0	0	4	0	239	4

Catchment area	Glomma

Food/drink related industry	0	0	0	0	13	0	0	13

Org. chemical/bio-chemical 	0	2	0	0	1	0	2	1

Other sectors	103	2	0	1	1	0	106	2

Pulp/paper industry	59	128	44	22	20	4	230	46

Waste processing industry	0	12	0	0	1	0	12	1

Sum	Glomma	162	144	44	23	36	4	350	63

Catchment area	Numedalslågen

Cookeries/refineries	0	0	0	6	0	0	0	6

Org. chemical/bio-chemical 	3	0	0	1	0	0	3	1

Other sectors	44	0	0	7	0	0	44	7

Pulp/paper industry	42	0	0	5	0	0	42	5

Waste processing industry	7	0	0	0	0	0	7	0

Sum	Numedalslågen	96	0	0	19	0	0	96	19

Catchment area	Orkla

Cookeries/refineries	1	0	0	0	0	0	1	0

Food/drink related industry	6	0	0	1	0	0	6	1

Other sectors	187	0	0	1	0	0	187	1

Pulp/paper industry	146	0	0	21	0	0	146	21

Sum	Orkla	340	0	0	23	0	0	340	23

Catchment area	Orreelva

Cookeries/refineries	1	0	0	0	0	0	1	0

Other sectors	37	0	0	1	0	0	37	1

Sum	Orreelva	38	0	0	1	0	0	38	1

Catchment area	Otra

Food/drink related industry	0	0	0	0	0	0	0	0

Org. chemical/bio-chemical 	4	0	0	0	0	0	4	0

Other sectors	0	0	0	0	0	0	0	0

Pulp/paper industry	4	25	0	2	1	0	29	3

Sum	Otra	8	25	0	2	1	0	33	3

Catchment area	Skienselva

Fertiliser industry	758	0	0	4	0	0	758	4

Food/drink related industry	0	4	0	0	1	0	4	1

Org. chemical/bio-chemical 	0	0	0	1	0	0	0	1

Other sectors	0	0	0	0	0	0	0	0

Pulp/paper industry	0	82	0	0	7	0	82	7

Sum	Skienselva	758	86	0	5	8	0	844	13

Catchment area	Suldalslågen

Cookeries/refineries	988	0	0	0	0	0	988	0

Food/drink related industry	0	0	0	0	0	0	0	1

Org. chemical/bio-chemical 	322	0	0	41	0	0	322	41

Other sectors	0	0	0	0	0	0	0	0

Waste processing industry	5	0	0	0	0	0	5	0

Sum	Suldalslågen	1315	0	0	41	0	0	1315	42

Catchment area	Vefsna

Food/drink related industry	0	0	0	0	0	0	0	0

Org. chemical/bio-chemical 	569	0	0	26	0	0	569	26

Other sectors	0	0	0	0	1	0	0	2

Waste processing industry	0	0	0	0	0	0	0	0

Sum	Vefsna	569	0	0	26	1	0	569	28

Sum	2000	3323	457	44	140	50	4	3824	196

Diffuse losses summary report (surface water) - national figures (tonnes/year)

Country	Normalised 	 Tot-N	Tot-P	Tot-N 	Tot-P	Tot- N	Tot-P	 Tot-N	 Tot-P	Tot-N	Tot-P

Year	or	Agric	Agric	Dir.  	 Dir. 	Other	 Other	Backgr	Backgr	 (Agric 	 (Agric +

Catchment 	dep.	dep.	+

Dir. dep 	 Dir. dep

+	+

Other -	 Other -

Norway

Data from	1985

Alta	Normalised	151	5	34	0	4318	228	4318	228	185	5

Alta	Year specific	126	5	28	0	3602	192	3602	192	154	5

Drammenselva	Normalised	2280	75	261	4	1952	80	1952	80	2541	79

Drammenselva	Year specific	2083	69	239	3	1783	81	1783	81	2322	72

Glomma	Year specific	9739	341	719	18	4719	264	4719	264	10458	359

Glomma	Normalised	12132	424	895	23	5878	310	5878	310	13027	447

Numedalslågen	Year specific	2244	75	80	3	979	36	979	36	2324	78

Numedalslågen	Normalised	2205	74	79	3	962	34	962	34	2284	77

Orkla	Normalised	6520	248	656	15	8550	278	8550	278	7176	263

Orkla	Year specific	6051	230	609	14	7935	264	7935	264	6660	244

Orreelva	Year specific	5035	175	2161	8	5849	130	5849	130	7196	183

Orreelva	Normalised	2801	97	1202	4	3255	61	3255	61	4003	101

Otra	Year specific	1071	31	888	2	4890	110	4890	110	1959	33

Otra	Normalised	911	26	756	1	4161	81	4161	81	1667	27

Skienselva	Year specific	747	22	616	2	2650	107	2650	107	1363	24

Skienselva	Normalised	703	21	580	2	2494	75	2494	75	1283	23

Suldalslågen	Year specific	3668	161	1777	5	16102	188	16102	188	5445	166

Suldalslågen	Normalised	2308	102	1118	3	10133	107	10133	107	3426	105

Vefsna	Year specific	1849	81	245	5	5486	258	5486	258	2094	86

Vefsna	Normalised	1535	67	204	4	4556	207	4556	207	1739	71

Sum	1985	64159	2329	13147	119	100254	3091	100254	3091	77306	2448

Data from	2000

Alta	Normalised	149	5	35	3	4318	228	4318	228	184	8

Alta	Year specific	167	6	39	3	4821	255	4821	255	206	9

Drammenselva	Normalised	1707	49	249	13	1938	80	1938	80	1956	62

Drammenselva	Year specific	2398	69	349	18	2723	112	2723	112	2747	87

Glomma	Year specific	13559	423	1254	60	8239	435	8239	435	14813	483

Glomma	Normalised	9552	298	884	42	5805	307	5805	307	10436	340

Numedalslågen	Year specific	2725	83	115	7	1315	48	1315	48	2840	90

Numedalslågen	Normalised	1904	58	81	5	919	34	919	34	1985	63

Orkla	Normalised	6603	246	676	21	12110	278	12110	278	7279	267

Orkla	Year specific	5630	210	577	18	10325	237	10325	237	6207	228

Orreelva	Year specific	3413	118	1513	20	4037	75	4037	75	4926	138

Orreelva	Normalised	2747	95	1218	16	3250	60	3250	60	3965	111

Otra	Year specific	1114	30	1109	21	6008	116	6008	116	2223	51

Otra	Normalised	772	21	768	15	4161	81	4161	81	1540	36

Skienselva	Year specific	835	22	884	43	3839	115	3839	115	1719	65

Skienselva	Normalised	542	14	574	28	2491	75	2491	75	1116	42

Suldalslågen	Year specific	2712	118	1334	17	11959	126	11959	126	4046	135

Suldalslågen	Normalised	2298	100	1131	14	10135	107	10135	107	3429	114

Vefsna	Year specific	1606	69	224	11	4752	219	4752	219	1830	80

Vefsna	Normalised	1523	66	212	11	4505	207	4505	207	1735	77

Sum	2000	61956	2100	13226	386	107650	3195	107650	3195	75182	2486

3. mars

| Statens forurensningstilsyn Postboks 8100 Dep, 0032 Oslo 1 |  |  |
| --- | --- | --- |
| Deres ref 1998/291 - 61 | Vår ref 1998/03392-18 | Dato 19.09.01 |

Rapportering til 5. Nordsjøkonferanse - Næringssalter.

Det vises til brev av 5.09.01 der det anmodes om rapportering på artikkel 36 i Esbjærg erklæringen. Rapporteringen skal være en kort omtale av utviklingen i arbeidet med å redusere tap av næringssalter fra landbruket.

Det er tatt utgangspunkt i utformingen av de 2 punktene i artikkel 36.

The Report No 64 to  Stortinget (1991-92) concerning Norway's implementation of the North Sea Declarations gave a comprehensive assessment of measures and instruments in the Agricultural sector. Integration of environmental concerns in the Agricultural policy has been re-examined twice during the last decade.

The report No 8 Agriculture in development (1992-93) to Stortinget established environmental principles for sustainable development.  The North Sea Declarations were considered as a basic challenge for reduction of water pollution. The report to  Stortinget No 19 (1999-2000) concerning Norwegian Agriculture and food production gave basis for more ecological oriented agricultural production and also a basis for introducing comprehensive environmental planning system on the farm level.

Combined use of legal and economic instruments supported with information and control has been an important strategy to reduce pollution. Monitoring of the discharges of nutrients from agricultural catchments has been run over a period of 15 years. A Result Control system is developed and run on a yearly basis to estimate and evaluate the effects of implemented measures to reduce losses of nutrients from agriculture. The system consists of three steps:

- Data collection and collocation are operated by Statistics of Norway.

- Modelling of discharge of nutrients from agriculture is carried out of Norwegian Centre for Soil and Environmental Research and The Norwegian Institute for Water Research.

- The evaluation of results and further administrative decisions and dimensions of measures necessary to achieve politically given goals are a part of the negotiations between the Government and the Farmers Unions in the Agreement of Agriculture. Thus assessment and adjustment of the policy and the use of instruments are done each year.

The reduction of N losses from agriculture has been calculated to 24 % during the period 1985 - 2000, whereas the reduction of P losses is 32 %. The relative reductions (in %) are related to the anthropogenic part of the agricultural nutrient losses. The loss estimates relate to normalised losses (long-term mean losses).

The reductions in the losses of N and P from agriculture have been estimated based on the results of different research projects and field investigations, and the date from Statistics Norway. Measures include soil tillage methods, catch crops, various aspects related to nutrient management (fertilisers and manure), drainage and surface water management, changes in crop composition and technical improvements of point sources in agriculture (silage and manure storages). Statistics Norway provides data on the regional implementation of various measures in agriculture. Detailed descriptions of methods and results are given in Eggestad & Vagstad, 2001: Losses of Nitrogen and Phosphorus from agriculture in the North Sea Convention area of Norway. Jordforsk Technical Report.

Further reductions in both N and P losses are possible. For nitrogen, the potentials are mainly associated with further increase in conservation tillage including the use of catch crops on arable land and more efficient use of nutrient in livestock feeding and plant production. Appropriate governmental instruments have already been established to enhance further decreases in the nutrient losses

The nutrient discharge estimates are linked directly to empirical data through calibration and validation against measured nutrient losses in several small agricultural catchments in different agro-ecological regions in South Eastern Norway. This ensures the reliability of the discharge estimates.

Med hilsen

Ragnar Mjelde									Eivind Berg

Underdirektør									Rådgiver

# Report from Sweden

2001-12-06

REPORT ON NUTRIENTS TO THE 5th NORTH SEA CONFERENCE

As clearly has been stated earlier, SEPA has during 2000 and 2001 run a project called TRK (Tranport, Retention och Källfördelning, ”Transport, Retention and Source Apportionment”) for the reporting of the HELCOM PLC 4 Project, and also RID 2000/5 th NSC. The Project is mainly aimed at investigation of diffuse sources.

The latest OSPAR EUC-meeting  however stressed the necessity of reporting the nutrient load to the Kattegat and Skagerrak earlier.

The report of Sweden is hereby submitted in the format (HARP-NUT) agreed upon.

The results of the investigation of the diffuse sources will be sent as soon as there is available an English version of the project.

e.o.

Anders Widell

Environmental Data Section			2001-12-19

Anders Widell

Secretariat 5th North Sea Conference		Comments to Draft Chapter Six

Reduction of Nutrient Inputs to the North Sea

Comments from Sweden.

The tables 2 to 6 have been scrutinized and compared to stated background documents, i.e HARP NUT Guidelines, duly filled in as requested with the exception of diffuse sources, as these have been extensively studied in a project, TRK, running in 2000 and 2001 with reporting date 2001-12-31, and earlier Swedish reports to OSPAR.

There are some data in these tables that must be changed and commented upon.

Table 2: seems to be OK.

Table 3: the figure 7200 tonnes for tot-N in 1985 should be compared to the given figure for 1987, 9200 tonnes. The following clarification should be inserted in “remarks”: The Swedish figure of 7200 tonnes tot-N from wwtp.s is considered low.  A more correct figure is 9000 tonnes, which corresponds better to the figure of 1987, 9200 tonnes. This gives a reduction rate of 32 %.

Clarification:

The figure was based on mostly estimated values from the wwtp.s, founded on the calculation 11g N/p,d and a reduction of 30 % in the average wwtp. Later investigations have shown that 12 g N/p,d and a reduction of 20 % in the average wwtp is more correct. If a correction factor of 12/11x0,8/07 is applied on the figure 7200 the result will be 9000.

The results of Sweden given in HARP tables are partly missing or put in the wrong columns in the following tables.

Table 4: first column 3288, second column 1995 (OK), third column –39 %, fourth column 216, fifth column 143, sixth column –34 %.

Table 5: first column 76, second column 62, third column –14 %, fourth column 10, fifth column 9 (OK), sixth column –10 %.

Table 6: first column 1630 (OK), second column 855 (OK), third column –48 %, fourth column 118, fifth column 88 (OK), sixth column –25 %.

Table 7: Please recalculate the figures of phosphorus. An average reduction of 0 % is not possible as the various reductions from tables 2 – 6 are 31 %, 36 %, 34 %, 10 % and 25 %. The reductions were taken at source!!!

Overall suggestion: As You already have stated that the figures from Sweden are from different years (but no explanations) there is no reason not to have them inserted to the tables. There should also be an explanation that some countries before 1985 had a lot of measures aimed at reduction of nutrients, mainly phosphorus, mostly in municipal treatment plants.

Environmental Data Section			2002-01-02

Anders Widell

Secretariat 5th North Sea Conference		Comments to Draft Chapter Six

Reduction of Nutrient Inputs to the North Sea

Comments from Sweden.

The tables 2 to 6 have been scrutinized and compared to stated background documents, i.e HARP NUT Guidelines, duly filled in as requested with the exception of diffuse sources, as these have been extensively studied in a project, TRK, running in 2000 and 2001 with reporting date 2001-12-31, and earlier Swedish reports to OSPAR.

There are some data in these tables that must be changed and commented upon.

Table 2: seems to be OK.

Table 3: The figures for wwtp.s should be:

for total N, year 1985: 9200, year 2000: 5446, reduction 41 %

for total P, year 1985: 262, year 2000: 157, reduction 40 %

References:

1985: Statistics Sweden, Report Na 22 SM 9901, covers the discharges in 1987, but the discharges in 1985 is not deemed to be smaller, rather larger, as further measures against the more apparent eutrophication problems started at that time.

2000: Statistics Sweden, Report MI XX SM 01YY, covers the discharges in 2000. Not yet published.

Table 4: The figures for households should be:

for total N, year 1985: 3288, year 2000: 1995, reduction 39 %

for total P, year 1985: 216, year 2000: 143, reduction 34 %

References:

1985 and 2000: Statistics over population connected and not connected to wwtp.s from Statistics Sweden, Reports Na 11 SM 9201 and Na 11 SM 9701.

Table 5: The figures for aquaculture should be:

for total N, year 1985: 76, year 2000: 62, reduction 19 %

for total P, year 1985: 10, year 2000: 9 , reduction 9 %

References: Statistics from Statistics Sweden, calculations from Swedish EPA.

Table 6: The figures for industries should be

for total N, year 1985: 1169, year 2000: 855, reduction 27 %

for total P, year 1985: 118, year 2000: 88, reduction 25 %

References: Reports from Swedish EPA, Report 5154 (Discharges from pulp- and paper industries in 2000), similar earlier reports and other compilations.

Table 7: The recalculated figures should be:

for total N, year 1985: 31390, year 2000: 17460, reduction 44 %

for total P, year 1985: 996, year 2000: 667, reduction 33 %

There should also be an explanation that some countries before 1985 had a lot of measures aimed at reduction of nutrients, mainly phosphorus, mostly in municipal treatment plants.

# Comments to the Swedish load figures as reported to 5th NSC

DIRECT DISCHARGES

Methods of measurement and calculation used:

### Municipal sewage water

The municipal sewage treatment plants in Sweden have different levels of sampling procedures depending on their sizes, as shown in the following table:

Table 1.

Size of treatment plant (pe)	Parameter	Frequency of

analyses

201	-	2 000	Tot-P, Tot-N	8 dp/year

201	-	2 000	BOD7 and CODCr	8 and 4 dp/year

2 001-	 10 000	Tot-P, Tot-N	2 dp/month

2 001-	 10 000	BOD7, CODCr	2 dp/month

10 001- 	20 000	Tot-P	2 dp/month

10 001-	 20 000	NH4-N, Tot-N	2 dp/month

10 001-	 20 000	BOD7, CODCr	2 dp/month

>	 20 000	Tot-P	1 wp/week

>	20 000	NH4-N, Tot-N	1 dp/week

>	20 000	BOD7	1 dp/week

>	20 000	CODCr	2 wp/month

>	20 000	Hg, Cd, Pb, Cu,

Zn, Cr and Ni	1 wp/month

dp = daily, continuous sampling proportional to the flow during 24 hrs.

wp = weekly, continuous sampling proportional to the flow

The calculation of the pollution load from the larger cities (>20 000 pe) is based upon at least 25 samples/year. The water flow through the treatment plants is measured continuosly. The pollution load is calculated as the product of annual flow and flow weighted concentration. Thus the reported pollution load from the municipal treatment plants is considered to be a fairly correct assumption of the true discharges.

The overflows of the larger municipalities are usually included in the figures given above.

The chemical analyses were performed in accordance to Nordic standard.

Tot-N and tot-P:		Potassium peroxodisulphate digestion, autoanalyzer.

Zn and Cu:		Graphite oven AAS.

Pb, Cd and Hg:	Graphite oven AAS.

#### Industrial effluents

Methods of measurement and calculation used, including information on the concentration upon which the measurement is based:

pulp- and  paper:	Tot P and tot-N, once a week.

Metals, once a month.

refinery   :	Metals, sampling four times a year for 5 days/week.

chemical plants:	P and N, random samples weekly.

The analyses were performed in accordance to Swedish and Nordic standards and have been described above.

## RIVERINE INPUTS

MAIN RIVERS AND SOME MINOR RIVERS

Methods of measurement and calculation used:

Monthly sampling for water chemistry analysis. Daily measurement of flow. Transport was calculated as daily Q x linear interpolation of concentrations.

N and P analysis as described above.

Cu and Zn graphite oven AAS.

Cd and Pb freeze drying (concentrating), graphite oven AAS.

The load from minor rivers and the coastal areas between the rivers are presented.

## MINOR RIVERS NOT MEASURED AND COASTAL AREAS

Methods of measurement and estimation used:

The estimations are based upon the monitoring results of the "neighbourhood" rivers. Weighted coefficients has been calculated for the different minor rivers and coastal areas reported

Catchment description summary report - national figures

Catchment name	Catchment 	Lake 	River 	Total 	Long term

Water 	Water 	Populatio

Area	Surface 	Surface	n	annual

flow

094/095 (67 %)	138	0	10100

094/095 - 107/108

095 Vege å	489	0	43000	160

095/096	1	0	0

096 Rönne å	1897	3	88100	741

096/097	153	0	12900

097 Stensån	286	3	7100	128

097/098	11	0	1900

097/098 - 107/108

098 Lagan	6452	598	118900	2705

098/099	2	0	0

099 Genevadsån	123	2	4700	128

099,100,102,104,106,107

099/100	9	0	1200

100 Fylleån	395	14	9000	237

100/101	6	0	6300

101 Nissan	2686	140	82800	1347

101/102	185	0	25300

102 Suseån	450	7	7600	234

102/103	17	0	4900

103 Ätran	3342	206	65600	1851

103/104	313	8	39300

104 Himleån	202	6	8200	120

104/105	38	0	1900

105 Viskan	2202	132	121200	1259

105/106	307	5	16000

106 Rolfsån	694	66	27100	376

106/107	0	0	0

107 Kungsbackaån	304	21	37400	150

107/108	240	2	126200

108 Göta älv	50117	9276	821900	18443

108, 108/109 - 112

108/109	1365	22	134400

108/109 - 111/112

109 Bäveån	301	18	21300	128

109/110	280	2	21600

110 Örekilsälven	1340	57	14500	748

110/111	955	3	40700

111 Strömsån	256	10	4900	142

111/112	157	3	3200

112 Enningdalsälven	782	66	3100	496

Sum	Sweden	76495	10670	1932300	29393

Retention in surface waters summary report - national figures (tonnes/year)

Country	Tot-N	Tot-N	Tot-N	Tot-P	Tot-P	Tot-P 	Tot-N	Tot-

Year	Coastal	Unmonitore	Monitored	Coastal	Unmonitore	Monitored 	 	P

Catchment 	area	d	inland	area	d	inland

inland	inland

Data from				(No submitted data)

Sum

Riverine load summary report - national figures (tonnes/year)

Country	Specific 	Riverine load,	Riverine load,	Riverine load,	Riverine load,	Transboundar	Transboundar

Year	annual 			 		y riverine 	y riverine

Catchment 	riverine flow	without 	without 	flow-	flow-	inputs, 	inputs,

normalisation	normalisation	normalised 	normalised

mill m3/year	 tonnes/yr 	 tonnes/yr 	tonnes/yr 	tonnes/yr 	tonnes/yr 	tonnes/yr

Sweden

Data from	2000

094/095 (67 %)	47	4	272

095 Vege å	233	11,8	905

095/096	0,02	1,9

096 Rönne å	725	45,9	2695

096/097	72	3,7	283

097 Stensån	4	6,9	530

097/098 - 107/108	662	19,8	1601

098 Lagan	2995	60,2	3105

099,100,102,104,106,107	2	35,4	2525

101 Nissan	1703	36,1	1574

103 Ätran	2239	45,4	2415

105 Viskan	1640	63,2	1773

108 Göta älv	23967	415	20355

108/109 - 111/112	2397	105	2453

109 Bäveån	252	11,1	250

110 Örekilsälven	1198	52,5	1224

13.f 2

Specific 	Riverine load,	Riverine load,	Riverine load,	Riverine load,	Transboundar	Transboundar

Year	annual 			 		y riverine 	y riverine

Catchment 	riverine flow	without 	without 	flow-	flow-	inputs, 	inputs,

normalisation	normalisation	normalised 	normalised

mill m3/year	 tonnes/yr 	 tonnes/yr 	tonnes/yr 	tonnes/yr 	tonnes/yr 	tonnes/yr

111 Strömsån	227	10	234

112 Enningdalsälven	725	9,2	507

Sum	2000	39088	935,22	42702,9 2 of 2

All anthropogenic sources - tonnes N

Country	WWTP	Industry	Aquaculture	Households	Diffuse 	Sum

Year	sources

Sweden

1995	0	0	0	1995	0	1995

1998	6194	0	0	0	0	6194

1999	0	0	61,5	0	0	61,5

2000	0	854,8	0	0	0	854,8

All anthropogenic sources - tonnes P

Country	WWTP	Industry	Aquaculture	Households	Diffuse 	Sum

Year	sources

Sweden

1995	0	0	0	143	0	143

1998	218	0	0	0	0	218

1999	0	0	9,1	0	0	9,1

2000	0	88,4	0	0	0	88,4

Aquaculture summary report - national figures (tonnes/year)

Country	Tot- N	Tot-N 	Tot- N	Tot-P 	Tot-P	Tot-P	Tot-N 	Tot-P

Year	Coastal 	Inland 	 Inland 	Coasta	 Inland 	Inland

Catchment 		monitore	unmonitore	l 	monitore	unmonitore

area

Sweden

Data from	1999

094/095 - 107/108	0	0	0	0	0	0	1,5	0,2

108, 108/109 - 112	0	0	0	0	0	0	60	8,9

Sum	1999	0	0	0	0	0	0	61,5	9,1

Sewage works and sewerage summary report - national figures (tonnes/year)

Country	Tot-N 	Tot-N	Tot-N	Tot-P	Tot- P	Tot-P	Tot-N 	Tot-P

Year	Coastal	 Inland	Inland	Coastal	 Inland 	 Inland

Catchment name	area	monitored	unmonitored	area	monitored	unmonitore

d

Sweden

Data from	1998

094/095 (67 %)	0	0	0	0	0	0	0	0

095 Vege å	0	69	0	0	1	0	69	1

095/096	0	0	0	0	0	0	0	0

096 Rönne å	3	122	0	51	0	0	125	51

096/097	23	0	0	0	0	0	23	0

097 Stensån	0	0	0	0	0	0	0	0

097/098	16	0	0	0	0	0	16	0

098 Lagan	12	228	0	1	3	0	240	4

098/099	0	0	0	0	0	0	0	0

099 Genevadsån	0	0	0	0	0	0	0	0

099/100	0	0	0	0	0	0	0	0

100 Fylleån	0	0	0	0	0	0	0	0

100/101	0	0	0	0	0	0	0	0

101 Nissan	117	132	0	5	1	0	249	6

101/102	9	0	0	1	0	0	9	1

102 Suseån	0	6	0	0	0	0	6	0

102/103	0	0	0	0	0	0	0	0

13. mars 2002	Page 1 of 2

Country	Tot-N 	Tot-N	Tot-N	Tot-P	Tot- P	Tot-P	Tot-N 	Tot-P

Year	Coastal	 Inland	Inland	Coastal	 Inland 	 Inland

Catchment name	area	monitored	unmonitored	area	monitored	unmonitore

d

103 Ätran	0	239	0	0	2	0	239	2

103/104	118	0	0	5	0	0	118	5

104 Himleån	0	0	0	0	0	0	0	0

104/105	0	0	0	0	0	0	0	0

105 Viskan	0	340	0	0	11	0	340	11

105/106	16	0	0	1	0	0	16	1

106 Rolfsån	0	5	0	0	0	0	5	0

106/107	0	0	0	0	0	0	0	0

107 Kungsbackaån	105	0	0	3	0	0	105	3

107/108	23	0	0	1	0	0	23	1

108 Göta älv	1712	2542	0	64	63	0	4254	127

108/109	111	10	0	2	0	0	121	2

109 Bäveån	93	0	0	1	0	0	93	1

109/110	9	0	0	0	0	0	9	0

110 Örekilsälven	0	10	0	0	0	0	10	0

110/111	102	12	0	2	0	0	114	2

111 Strömsån	0	0	0	0	0	0	0	0

111/112	10	0	0	0	0	0	10	0

112 Enningdalsälven	0	0	0	0	0	0	0	0

Sum	1998	2479	3715	0	137	81	0	6194	218

Households not connected to public sewerage summary report - national figures

Country	Tot-N 	Tot-N	Tot-N	Tot-P	Tot-P	Tot-P	Tot-N 	Tot-P

Year	Coastal 	 Inland	Inland	Coastal 	 Inland 	 Inland

Catchment name	area	monitored	unmonitored	area 	monitored	unmonitore

d

Sweden

Data from	1995

094/095 (67 %)	0	0	0	0	0	0	14	1,1

095 Vege å	0	0	0	0	0	0	35	2,7

095/096	0	0	0	0	0	0	0	0

096 Rönne å	0	0	0	0	0	0	112	8,4

096/097	0	0	0	0	0	0	17	1,3

097 Stensån	0	0	0	0	0	0	19	1,3

097/098	0	0	0	0	0	0	0	0

098 Lagan	0	0	0	0	0	0	190	13,6

098/099	0	0	0	0	0	0	0	0

099 Genevadsån	0	0	0	0	0	0	9	0,7

099/100	0	0	0	0	0	0	1	0,1

100 Fylleån	0	0	0	0	0	0	15	1,1

100/101	0	0	0	0	0	0	0	0

101 Nissan	0	0	0	0	0	0	74	5,3

101/102	0	0	0	0	0	0	12	0,9

102 Suseån	0	0	0	0	0	0	20	1,4

102/103	0	0	0	0	0	0	1	0,1

103 Ätran	0	0	0	0	0	0	120	8,4

103/104	0	0	0	0	0	0	30	2,1

104 Himleån	0	0	0	0	0	0	22	1,5

104/105	0	0	0	0	0	0	8	0,6

105 Viskan	0	0	0	0	0	0	139	9,7

105/106	0	0	0	0	0	0	43	3,1

106 Rolfsån	0	0	0	0	0	0	62	4,3

106/107	0	0	0	0	0	0	0	0

107 Kungsbackaån	0	0	0	0	0	0	41	3

107/108	0	0	0	0	0	0	29	2,1

108 Göta älv	0	0	0	0	0	0	523	37,4

108/109	0	0	0	0	0	0	237	17,1

109 Bäveån	0	0	0	0	0	0	23	1,6

109/110	0	0	0	0	0	0	41	2,9

110 Örekilsälven	0	0	0	0	0	0	45	3,2

110/111	0	0	0	0	0	0	80	5,7

111 Strömsån	0	0	0	0	0	0	9	0,6

111/112	0	0	0	0	0	0	9	0,6

112 Enningdalsälven	0	0	0	0	0	0	15	1,1

Sum	1995	0	0	0	0	0	0	1995	143

Industrial plants report - national figures

(tonnes/year)

Country	Tot-N 	Tot-N	Tot-N	Tot-P	Tot-P	Tot-P	Tot-N 	Tot-P

Year	Coastal	 Inland	Inland	Coastal	 Inland 	 Inland

Catchment 	area	monitored	unmonitored	area	monitored	unmonitore

name	d

Industry

Sweden

Data from	2000

Catchment area	096 Rönne å

Pulp/paper industry	0	9	0	0	0,2	0	9	0,2

Sum	096 Rönne å	0	9	0	0	0,2	0	9	0,2

Catchment area	098 Lagan

Pulp/paper industry	0	11	0	0	0,7	0	11	0,7

Sum	098 Lagan	0	11	0	0	0,7	0	11	0,7

Catchment area	101 Nissan

Pulp/paper industry	0	28	0	0	2,8	0	28	2,8

Sum	101 Nissan	0	28	0	0	2,8	0	28	2,8

Catchment area	105/106

Pulp/paper industry	101	0	0	22	0	0	101	22

Sum	105/106	101	0	0	22	0	0	101	22

Country	Tot-N 	Tot-N	Tot-N	Tot-P	Tot-P	Tot-P	Tot-N 	Tot-P

Year	Coastal	 Inland	Inland	Coastal	 Inland 	 Inland

Catchment 	area	monitored	unmonitored	area	monitored	unmonitore

name	d

Industry

Catchment area	108 Göta älv

Pulp/paper industry	0	620	0	0	58	0	620	58

Sum	108 Göta älv	0	620	0	0	58	0	620	58

Catchment area	108/109

Cookeries/refineries	22,6	0	0	0,4	0	0	22,6	0,4

Org. chemical/bio-chemical 	53,2	0	0	3,7	0	0	53,2	3,7

Sum	108/109	75,8	0	0	4,1	0	0	75,8	4,1

Catchment area	110/111

Cookeries/refineries	10	0	0	0,6	0	0	10	0,6

Sum	110/111	10	0	0	0,6	0	0	10	0,6

Sum	2000	186,8	668	0	26,7	61,7	0	854,8	88,4

Diffuse losses summary report (surface water) - national figures (tonnes/year)

Country	Normalised 	 Tot-N	Tot-P	Tot-N 	Tot-P	Tot- N	Tot-P	 Tot-N	 Tot-P	Tot-N	Tot-P

Year	or	Agric	Agric	Dir.  	 Dir. 	Other	 Other	Backgr	Backgr	 (Agric 	 (Agric +

Catchment 	dep.	dep.	+

Dir. dep 	 Dir. dep

+	+

Other -	 Other -

Data from			(Figures submitted at an earlier stage have been used)

Sum

# Report from Switzerland

Catchment description summary report - national figures

Catchment name	Catchment 	Lake 	River 	Total 	Long term

Water 	Water 	Populatio

Area	Surface 	Surface	n	annual

flow

Rhine downstream lakes	9500	0	0,86	3000000	32324

Sum	Switzerland	9500	0	0,86	3000000	32324

Retention in surface waters summary report - national figures (tonnes/year)

Country	Tot-N	Tot-N	Tot-N	Tot-P	Tot-P	Tot-P 	Tot-N	Tot-

Year	Coastal	Unmonitore	Monitored	Coastal	Unmonitore	Monitored 	 	P

Catchment 	area	d	inland	area	d	inland

inland	inland

Data from				(No submitted data)

Sum

Riverine load summary report - national figures (tonnes/year)

Country	Specific 	Riverine load,	Riverine load,	Riverine load,	Riverine load,	Transboundar	Transboundar

Year	annual 			 		y riverine 	y riverine

Catchment 	riverine flow	without 	without 	flow-	flow-	inputs, 	inputs,

normalisation	normalisation	normalised 	normalised

mill m3/year	 tonnes/yr 	 tonnes/yr 	tonnes/yr 	tonnes/yr 	tonnes/yr 	tonnes/yr

Data from						(No submitted data)

Sum

All anthropogenic sources - tonnes N

Country	WWTP	Industry	Aquaculture	Households	Diffuse 	Sum

Year	sources

Switzerland

1985	18000	1000	0	1000	1886	21886

2000	12300	800	30	100	8444	21674

All anthropogenic sources - tonnes P

Country	WWTP	Industry	Aquaculture	Households	Diffuse 	Sum

Year	sources

Switzerland

1985	2300	153	0	250	320	3023

2000	900	20	3	10	147	1080

Aquaculture summary report - national figures (tonnes/year)

Country	Tot- N	Tot-N 	Tot- N	Tot-P 	Tot-P	Tot-P	Tot-N 	Tot-P

Year	Coastal 	Inland 	 Inland 	Coasta	 Inland 	Inland

Catchment 		monitore	unmonitore	l 	monitore	unmonitore

area

Switzerland

Data from	2000

Rhine downstream lakes	0	30	0	0	3	0	30	3

Sum	2000	0	30	0	0	3	0	30	3

Sewage works and sewerage summary report - national figures (tonnes/year)

Country	Tot-N 	Tot-N	Tot-N	Tot-P	Tot- P	Tot-P	Tot-N 	Tot-P

Year	Coastal	 Inland	Inland	Coastal	 Inland 	 Inland

Catchment name	area	monitored	unmonitored	area	monitored	unmonitore

d

Switzerland

Data from	1985

Rhine downstream lakes	0	18000	0	0	2300	0	18000	2300

Sum	1985	0	18000	0	0	2300	0	18000	2300

Data from	2000

Rhine downstream lakes	0	12300	0	0	900	0	12300	900

Sum	2000	0	12300	0	0	900	0	12300	900

Households not connected to public sewerage summary report - national figures

Country	Tot-N 	Tot-N	Tot-N	Tot-P	Tot-P	Tot-P	Tot-N 	Tot-P

Year	Coastal 	 Inland	Inland	Coastal 	 Inland 	 Inland

Catchment name	area	monitored	unmonitored	area 	monitored	unmonitore

d

Switzerland

Data from	1985

Rhine downstream lakes	0	1000	0	0	250	0	1000	250

Sum	1985	0	1000	0	0	250	0	1000	250

Data from	2000

Rhine downstream lakes	0	100	0	0	10	0	100	10

Sum	2000	0	100	0	0	10	0	100	10

Industrial plants report - national figures (tonnes/year)

Country	Tot-N 	Tot-N	Tot-N	Tot-P	Tot-P	Tot-P	Tot-N 	Tot-P

Year	Coastal	 Inland	Inland	Coastal	 Inland 	 Inland

Catchment 	area	monitored	unmonitored	area	monitored	unmonitore

name	d

Industry

Switzerland

Data from	1985

Catchment area	Rhine downstream lakes

0	1000	0	0	153	0	1000	153

Sum	Rhine downstream 	0	1000	0	0	153	0	1000	153

Sum	1985	0	1000	0	0	153	0	1000	153

Data from	2000

Catchment area	Rhine downstream lakes

0	800	0	0	20	0	800	20

Sum	Rhine downstream 	0	800	0	0	20	0	800	20

Sum	2000	0	800	0	0	20	0	800	20

Diffuse losses summary report (surface water) - national figures (tonnes/year)

Country	Normalised 	 Tot-N	Tot-P	Tot-N 	Tot-P	Tot- N	Tot-P	 Tot-N	 Tot-P	Tot-N	Tot-P

Year	or	Agric	Agric	Dir.  	 Dir. 	Other	 Other	Backgr	Backgr	 (Agric 	 (Agric +

Catchment 	dep.	dep.	+

Dir. dep 	 Dir. dep

+	+

Other -	 Other -

Switzerland

Data from	1985

Rhine downstream lakes	Normalised	10802	408	206	6	604	4	9726	98	1886	320

Sum	1985	10802	408	206	6	604	4	9726	98	1886	320

Data from	2000

Rhine downstream lakes	Normalised	11446	272	162	5	1565	8	4729	138	8444	147

Sum	2000	11446	272	162	5	1565	8	4729	138	8444	147

# Swiss Delegation in CONSSO	Bern, 20 December 2002

## 5NSC Progress Report Chap. 6

Nutrients – Total of all anthropogenic sources, background load and overall total

(the same figures are used for different sources as submitted in the reporting format!)

| Switzerland Nitrogen (tonnes N/year) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 1985 | % | 2000 | % |
| WWTP | 18‘000 | 57 | 12‘300 | 47 |
| Industry | 1000 | 3 | 800 | 3 |
| Aquaculture | --- |  | 30 | 0 |
| Hauseholds | 1000 | 3 | 100 | 0 |
| # Diffuse Sources | 11‘612 | 37 | 13‘173 | 50 |
| Total anthropogenic sources | 31‘612 | 100 | 26‘403 | 100 |
| Background Load | 9726 |  | 4729 |  |
| Overall TOTAL | 41‘338 |  | 31‘132 |  |

| Switzerland Phosphorus (tonnes P/year) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 1985 | % | 2000 | % |
| WWTP | 2300 | 74 | 900 | 74 |
| Industry | 153 | 5 | 20 | 2 |
| Aquaculture | --- |  | 3 | 0 |
| Hauseholds | 250 | 8 | 10 | 1 |
| # Diffuse Sources | 418 | 13 | 285 | 23 |
| Total anthropogenic sources | 3121 | 100 | 1218 | 100 |
| Background Load | 98 |  | 138 |  |
| Overall TOTAL | 3219 |  | 1356 |  |

# Report from the Netherlands

Catchment description summary report – national figures

Catchment name	Catchment 	Lake 	River 	Total 	Long term

Water 	Water 	Populatio

Area	Surface 	Surface	n	annual

flow

Ems	0	3,78	0	557270	0

Meuse	0	19,2	253,54	3678119	24756

Rhine	0	2253,18	176,62	11140738	42101

Scheldt	0	153,59	1,9	384073	4005

Sum	Netherlands	0	2429,75	432,06	15760200	70862

Retention in surface waters summary report – national figures (tonnes/year)

Country	Tot-N	Tot-N	Tot-N	Tot-P	Tot-P	Tot-P 	Tot-N	Tot-

Year	Coastal	Unmonitore	Monitored	Coastal	Unmonitore	Monitored 	 	P

Catchment 	area	d	inland	area	d	inland

inland	inland

Data from				(No information)

Sum

Riverine load summary report - national figures (tonnes/year)

Country	Specific 	Riverine load,	Riverine load,	Riverine load,	Riverine load,	Transboundar	Transboundar

Year	annual 			 		y riverine 	y riverine

Catchment 	riverine flow	without 	without 	flow-	flow-	inputs, 	inputs,

normalisation	normalisation	normalised 	normalised

mill m3/year	 tonnes/yr 	 tonnes/yr 	tonnes/yr 	tonnes/yr 	tonnes/yr 	tonnes/yr

Netherlands

Data from	1985

Ems	0	0	0	0	0	0	0

Meuse	0	0	0	0	0	3100	28500

Rhine	0	33000	400000	0	0	36200	392000

Scheldt	0	4000	34500	0	0	4000	34500

Sum	1985	0	37000	434500	0	0	43300	455000

Data from	1999

Ems	0	0	0	0	0	0	0

Meuse	0	4200	111000	0	0	3640	38560

Rhine	0	9800	145000	0	0	18700	300700

Scheldt	0	2200	33600	0	0	2200	33600

Sum	1999	0	16200	289600	0	0	24540	372860

All anthropogenic sources nitrogen (tonnes/year)

Country	WWTP	Industry	Aquaculture	Households	Diffuse 	Sum

Year	sources

Netherlands

1985	38410	19529	0	8481	10403000	10469420

1999	31000	4631,3	0	665	6075264	6111560,3

All anthropogenic sources phosphorous (tonnes/year)

Country	WWTP	Industry	Aquaculture	Households	Diffuse 	Sum

Year	sources

Netherlands

1985	10800	13422	0	1773	820000	845995

1999	3000	3383,7	0	72	470000	476455,7

Aquaculture summary report

Country	Tot- N	Tot-N 	Tot- N	Tot-P 	Tot-P	Tot-P	Tot-N 	Tot-P

Year	Coastal 	Inland 	 Inland 	Coasta	 Inland 	Inland

Catchment 		monitore	unmonitore	l 	monitore	unmonitore

area

Data from						(No submitted data)

Sum

Sewage works and sewerage summary report (tonnes/year)

Country	Tot-N 	Tot-N	Tot-N	Tot-P	Tot- P	Tot-P	Tot-N 	Tot-P

Year	Coastal	 Inland	Inland	Coastal	 Inland 	 Inland

Catchment name	area	monitored	unmonitored	area	monitored	unmonitore

d

Netherlands

Data from	1985

Ems	0	0	1550	0	0	330	1550	330

Meuse	0	8970	0	0	2530	0	8970	2530

Rhine	3890	23180	0	80	7610	0	27070	7690

Scheldt	0	0	820	0	0	250	820	250

Sum	1985	3890	32150	2370	80	10140	580	38410	10800

Data from	1999

Ems	0	0	1130	0	0	60	1130	60

Meuse	0	6890	0	0	730	0	6890	730

Rhine	3890	17270	0	140	1900	0	21120	2040

Scheldt	0	0	1860	0	0	170	1860	170

Sum	1999	3890	24160	2990	140	2630	230	31000	3000

Households not connected to public sewerage summary report - national figures (tonnes/year)

Country	Tot-N 	Tot-N	Tot-N	Tot-P	Tot-P	Tot-P	Tot-N 	Tot-P

Year	Coastal 	 Inland	Inland	Coastal 	 Inland 	 Inland

Catchment name	area	monitored	unmonitored	area 	monitored	unmonitore

d

Netherlands

Data from	1985

Ems	0	0	300	0	0	63	300	63

Meuse	0	1979	0	0	414	0	1979	414

Rhine	0	5995	0	0	1253	0	5995	1253

Scheldt	0	0	207	0	0	43	207	43

Sum	1985	0	7974	507	0	1667	106	8481	1773

Data from	1999

Ems	0	0	24	0	0	3	24	3

Meuse	0	155	0	0	17	0	155	17

Rhine	0	470	0	0	50	0	470	50

Scheldt	0	0	16	0	0	2	16	2

Sum	1999	0	625	40	0	67	5	665	72

Country	Tot-N 	Tot-N	Tot-N	Tot-P	Tot-P	Tot-P	Tot-N 	Tot-P

Year	Coastal	 Inland	Inland	Coastal	 Inland 	 Inland

Catchment 	area	monitored	unmonitored	area	monitored	unmonitore

name	d

Industry

Netherlands

Data from	1985

Catchment area	Ems

Food/drink related industry	0	0	3404	0	0	262	3404	262

Org. chemical/bio-chemical 	0	0	1201	0	0	3	1201	3

Other sectors	0	0	284	0	0	2	284	2

Pulp/paper industry	0	0	240	0	0	0,4	240	0,4

Sum	Ems	0	0	5129	0	0	267,4	5129	267,4

Catchment area	Meuse

Food/drink related industry	0	141	0	0	14	0	141	14

Org. chemical/bio-chemical 	0	447	0	0	7	0	447	7

Other sectors	0	2342	0	0	86	0	2342	86

Pulp/paper industry	0	136	0	0	0,2	0	136	0,2

Sum	Meuse	0	3066	0	0	107,2	0	3066	107,2

Catchment area	Rhine

Cookeries/refineries	0	715	0	0	55	0	715	55

13. mars 2002	Page 1 of 4

Country	Tot-N 	Tot-N	Tot-N	Tot-P	Tot-P	Tot-P	Tot-N 	Tot-P

Year	Coastal	 Inland	Inland	Coastal	 Inland 	 Inland

Catchment 	area	monitored	unmonitored	area	monitored	unmonitore

name	d

Industry

Fertiliser industry	0	2244	0	0	5127	0	2244	5127

Food/drink related industry	112	725	0	29	317	0	837	346

Org. chemical/bio-chemical 	0	1802	0	0	173	0	1802	173

Other sectors	1621	1660	0	9	6813	0	3281	6821

Pulp/paper industry	0	686	0	0	26	0	686	26

Waste processing industry	0	77	0	0	0,4	0	77	0,4

Sum	Rhine	1733	7909	0	38	12511,4	0	9642	12548,4

Catchment area	Scheldt

Cookeries/refineries	0	0	91	0	0	0	91	0

Fertiliser industry	0	0	333	0	0	83	333	83

Food/drink related industry	0	0	122	0	0	31	122	31

Org. chemical/bio-chemical 	0	0	471	0	0	23	471	23

Other sectors	0	0	604	0	0	361	604	361

Pulp/paper industry	0	0	71	0	0	1	71	1

Sum	Scheldt	0	0	1692	0	0	499	1692	499

Sum	1985	1733	10975	6821	38	12618,6	766,4	19529	13422

Data from	1999

Catchment area	Ems

Food/drink related industry	0	0	43	0	0	6	43	6

Org. chemical/bio-chemical 	0	0	109	0	0	0,1	109	0,1

Other sectors	0	0	49	0	0	1	49	1

Pulp/paper industry	0	0	38	0	0	12	38	12

Sum	Ems	0	0	239	0	0	19,1	239	19,1

13. mars 2002	Page2 of 4

Industrial plants report – National figures (tonnes/year)

Country	Tot-N 	Tot-N	Tot-N	Tot-P	Tot-P	Tot-P	Tot-N 	Tot-P

Year	Coastal	 Inland	Inland	Coastal	 Inland 	 Inland

Catchment 	area	monitored	unmonitored	area	monitored	unmonitore

name	d

Industry

Catchment area	Meuse

Food/drink related industry	0	77	0	0	56	0	77	56

Org. chemical/bio-chemical 	0	21	0	0	6	0	21	6

Other sectors	0	731	0	0	16	0	731	16

Pulp/paper industry	0	36	0	0	4	0	36	4

Sum	Meuse	0	865	0	0	82	0	865	82

Catchment area	Rhine

Cookeries/refineries	0	445	0	0	5	0	445	5

Fertiliser industry	0	868	0	0	1974	0	868	1974

Food/drink related industry	30	213	0	19	136	0	243	155

Org. chemical/bio-chemical 	0	306	0	0	19	0	306	19

Other sectors	831	376	0	20	951	0	1208	970

Pulp/paper industry	2	100	0	0,3	21	0	102	21,3

Waste processing industry	0	64	0	0	0,3	0	64	0,3

Sum	Rhine	863	2372	0	39,3	3106,3	0	3236	3144,6

Catchment area	Scheldt

Cookeries/refineries	0	0	0,3	0	0	0	0,3	0

Fertiliser industry	0	0	61	0	0	16	61	16

Food/drink related industry	0	0	110	0	0	87	110	87

Org. chemical/bio-chemical 	0	0	58	0	0	3	58	3

Other sectors	0	0	17	0	0	15	17	15

Waste processing industry	0	0	45	0	0	17	45	17

13. mars 2002	Page 3 of 4

Country	Tot-N 	Tot-N	Tot-N	Tot-P	Tot-P	Tot-P	Tot-N 	Tot-P

Year	Coastal	 Inland	Inland	Coastal	 Inland 	 Inland

Catchment 	area	monitored	unmonitored	area	monitored	unmonitore

name	d

Industry

Sum	Scheldt	0	0	291,3	0	0	138	291,3	138

Sum	1999	863	3237	530,3	39,3	3188,3	157,1	4631,3	3383,7

Diffuse losses summary report  (surface water) – national figures (tonnes/year))

Country	Normalised 	 Tot-N	Tot-P	Tot-N 	Tot-P	Tot- N	Tot-P	 Tot-N	 Tot-P	Tot-N	Tot-P

Year	or	Agric	Agric	Dir.  	 Dir. 	Other	 Other	Backgr	Backgr	 (Agric 	 (Agric +

Catchment 	dep.	dep.	+

Dir. dep 	 Dir. dep

+	+

Other -	 Other -

Data from	1985

Ems	Normalised	519000	41000	106	0	0	0	0	0	519106	41000

Meuse	Normalised	2076000	164000	2183	0	0	0	0	0	2078183	164000

Rhine	Normalised	7266000	574000	19465	0	0	0	0	0	7285465	574000

Scheldt	Normalised	519000	41000	1246	0	0	0	0	0	520246	41000

Sum	1985	10380000	820000	23000	0	0	0	0	0	10403000	820000

Data from	1999

Ems	Normalised	303500	23500	24	0	0	0	0	0	303524	23500

Meuse	Normalised	1214000	94000	500	0	0	0	0	0	1214500	94000

Rhine	Normalised	4249000	329000	4455	0	0	0	0	0	4253455	329000

Scheldt	Normalised	303500	23500	285	0	0	0	0	0	303785	23500

Sum	1999	6070000	470000	5264	0	0	0	0	0	6075264	470000

Directive 91/271/EC on waste water treatment

Inhabitant = the equivalent to the average daily pollution produced by one person per day.